GroupTrack

Master Config Reference — v1

Captured April 2, 2026  |  Device: tracker-t1000-e  |  Firmware: 2.6.11.60ec05e  |  configVersion: 2

## Hardware & Firmware

| Field | Value | Notes |
| --- | --- | --- |
| hardwareModel | tracker-t1000-e | Seeed Studio T1000-E tracker |
| firmwareVersion | 2.6.11.60ec05e | Current firmware version |
| pioEnv | tracker-t1000-e | PlatformIO environment |
| longName | !0d09dd28 | Node long name — change per device |
| shortName |  | Short name — blank uses last 4 of node ID |
| capturedDate | 2026-04-02 | Date master config was captured |
| capturedFirmware | 2.6.11.60ec05e | Firmware at capture time |
| configVersion | 2 | Version check — forces restore if device file is older |

## Device Settings

| Field | Value | Notes |
| --- | --- | --- |
| nodeRole | TRACKER | TRACKER role — optimized for position broadcasting |
| isManaged | false | Not remotely managed |
| serialEnabled | false | Serial module disabled |

## LoRa Radio Settings

| Field | Value | Notes |
| --- | --- | --- |
| loraRegion | US | US 915MHz band |
| loraModemPreset | LONG_FAST | Long range, faster data rate preset |
| loraBandwidth | 0 | 0 = implied by LONG_FAST preset |
| loraSpreadFactor | 0 | 0 = implied by LONG_FAST preset |
| loraCodingRate | 0 | 0 = implied by LONG_FAST preset |
| loraHopLimit | 3 | Max 3 mesh hops |
| loraTxEnabled | true | Transmit enabled |
| loraTxPower | 27 | 27 dBm transmit power (~500mW) |
| loraChannelNum | 5 | Channel 5 — 903MHz center frequency |

## Channel & Encryption

| Field | Value | Notes |
| --- | --- | --- |
| primaryChannelName | Convoy | GroupTrack master channel name |
| primaryChannelPsk | MEJHR0hQQVliRFpVbDAwSW1ocUF5d2Y2S0lobExCaFo= | Base64 AES256 encryption key — KEEP SECRET |
| channelId | 0 | Primary channel index 0 |
| channelUplinkEnabled | false | MQTT uplink disabled |
| channelDownlinkEnabled | false | MQTT downlink disabled |
| channelIsMuted | false | Channel not muted |
| positionPrecision | 32 | Full GPS precision |

## GPS & Position

| Field | Value | Notes |
| --- | --- | --- |
| gpsEnabled | true | GPS enabled |
| gpsMode | ENABLED | GPS always on |
| gpsUpdateSecs | 1 | GPS fix update every 1 second |
| gpsAttemptTime | 0 | 0 = no timeout on GPS acquisition |
| positionBroadcastSecs | 5 | Broadcast position every 5 seconds |
| smartPositionEnabled | true | Smart position — broadcast on significant movement |
| fixedPosition | false | Not a fixed position node |
| positionFlags | 811 | Position fields included in broadcast |
| smartMinIntervalSecs | 5 | Minimum 5 seconds between smart broadcasts |
| smartMinDistanceMeters | 20 | Minimum 20 meters movement to trigger broadcast |

## Display Settings

| Field | Value | Notes |
| --- | --- | --- |
| displayUnits | 0 | 0 = metric |
| screenTimeout | 600 | Screen off after 600 seconds (10 min) |
| autoScreenBrightness | false | Manual brightness |
| compassNorthTop | false | Compass heading-up mode |

## Telemetry

| Field | Value | Notes |
| --- | --- | --- |
| telemetryDeviceInterval | 0 | 0 = use firmware default |
| telemetryEnvInterval | 0 | 0 = use firmware default |
| telemetryEnvEnabled | false | Environment sensors disabled |

## Modules — All Disabled

| Field | Value | Notes |
| --- | --- | --- |
| mqttEnabled | false | MQTT disabled — off-grid only |
| mqttAddress |  | No MQTT broker |
| mqttUsername |  | No MQTT credentials |
| mqttEncryptionEnabled | false |  |
| mqttJsonEnabled | false |  |
| serialModuleEnabled | false | Serial module off |
| serialBaud | 0 |  |
| extNotificationEnabled | false | External notification off |
| extNotificationAlertMsg | false |  |
| rangeTestEnabled | false | Range test off |
| storeForwardEnabled | false | Store & forward off |
| neighborInfoEnabled | false | Neighbor info off |
| detectionSensorEnabled | false | Detection sensor off |
| audioEnabled | false | Audio off |
| deviceProfileBase64 |  | No device profile override |

GroupTrack  |  Master Config Reference v1  |  April 3, 2026  |  configVersion: 2