Test Plan

Meshtastic Off-Grid Convoy Tracking System

| Version | v2.0 |
| --- | --- |
| Date | March 2026 |
| Location | New Harmony, Utah (37.4794°N, 113.3097°W) |
| Author | Convoy System Team |
| Status | ACTIVE — Gate 1 Validation Pending |
| Coverage | Gate 1 GeoJSON Overlay · Gate 2 Tile Sharing · Gate 3 Tracks Only · Backend · Overlay UI · Field |

# 1. Overview

This test plan covers the complete validation sequence for the Meshtastic Convoy Tracking System from initial local server verification through live field deployment. Tests are organized into seven suites aligned with the dual-path architecture decision: Suite A validates the primary GeoJSON overlay path; Suites B1 and B2 cover the fallback paths; remaining suites cover the shared backend, overlay UI, and full field operation.

Each test case includes explicit pass criteria, fail actions, and a result block for field completion. Tests must be executed in the sequence shown — each suite's entry criteria depends on the prior suite's results. The Gate 1 result (TC-A-01 through TC-A-10) is the single most important outcome in this document and determines the entire deployment architecture.

## 1.1 Test Environment

| Backend host | MacOS/Linux laptop running Python 3.9+ |
| --- | --- |
| Backend port | 8765 (default) |
| Phone | iOS 16+ or Android 11+ — Gaia GPS or onX installed |
| Network | Laptop WiFi hotspot — phone connected as client |
| Simulation data | New Harmony, Utah — 20 nodes, 37.4794°N 113.3097°W, NW route |
| Meshtastic HW | Required for suites TC-E and TC-F only |
| Gaia GPS | Version 2024+ — Custom Map Sources feature required |
| onX Hunt/Offroad | Any current version — Custom Layers feature required |
| Browser | Safari (iOS) or Chrome (Android) — for overlay testing |

## 1.2 Test Suite Summary

| Suite | Name | Cases | Priority | Notes |
| --- | --- | --- | --- | --- |
| TC-A | Gate 1: GeoJSON Overlay (Path A) | 10 | CRITICAL | Determines primary architecture. Must run first. |
| TC-B | Gate 2: Offline Map Sharing (Path B1) | 6 | HIGH | Run only if TC-A fails. Determines fallback path. |
| TC-C | Gate 3: Tracks Only (Path B2) | 4 | MEDIUM | Always viable. Baseline capability confirmation. |
| TC-D | Backend Server | 8 | CRITICAL | Common to all paths. Run in parallel with TC-A. |
| TC-E | Overlay HUD | 9 | HIGH | Path A UI validation. Run after TC-A passes. |
| TC-F | Live Meshtastic Integration | 7 | CRITICAL | Live hardware. Run after TC-D passes. |
| TC-G | Full Field Operation | 8 | CRITICAL | All systems, real vehicles. Run after TC-E + TC-F. |
|  | TOTAL | 52 |  |  |

## 1.3 Priority Definitions

| CRITICAL | Test must pass before field deployment. Failure blocks all further testing in that path. |
| --- | --- |
| HIGH | Test should pass. Failure triggers fallback path evaluation. |
| MEDIUM | Important but non-blocking. Document and schedule fix. |
| LOW | Nice to have. Can be deferred post-field. |

# Suite A — Gate 1: GeoJSON Overlay (Path A)

ENTRY CRITERIA: Laptop running Python 3.9+. Phone with Gaia GPS installed. Both on same WiFi or laptop hotspot.

GATE DECISION: If TC-A-01 through TC-A-08 all PASS → Path A confirmed. Proceed to TC-E and TC-F. Skip TC-B and TC-C.

If any of TC-A-01 through TC-A-07 FAIL → Document failure mode. Proceed to Suite TC-B (Gate 2).

TC-A-01 GeoJSON Server Starts and Prints LAN IP Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | Python 3.9+ installed. geojson_server.py present in working directory. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open terminal on laptop | Terminal prompt available | □ |
| 2 | Run: python3 geojson_server.py | No import errors. Server starts. | □ |
| 3 | Check output for LAN IP line | Line reads: 'LAN IP detected: 192.168.x.x' | □ |
| 4 | Check output for GeoJSON URL line | Line reads: 'Gaia/onX URL: http://192.168.x.x:8765/convoy.geojson' | □ |
| 5 | Check output for 'Serving on port 8765' | Port confirmed in startup output | □ |
| 6 | Record the LAN IP for subsequent tests | IP noted for use in TC-A-04 through TC-A-10 | □ |

| PASS if: Server starts without errors. LAN IP detected and printed. GeoJSON URL printed. | On FAIL: Check Python version (python3 --version). Check for missing dependencies. Verify file path. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-02 GeoJSON Endpoint Returns Valid Response — Local Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-01 passed. Terminal with server running. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open second terminal | New terminal prompt | □ |
| 2 | Run: curl -s http://localhost:8765/convoy.geojson ¦ python3 -m json.tool ¦ head -20 | JSON output begins printing | □ |
| 3 | Verify first line is: { | Output starts with opening brace | □ |
| 4 | Verify line: "type": "FeatureCollection" | Type field present and correct | □ |
| 5 | Verify "features" array present | "features": [ | □ |
| 6 | Count features in output: python3 -c "import json,urllib.request; d=json.loads(urllib.request.urlopen('http://localhost:8765/convoy.geojson').read()); print(len(d['features']))" | Prints: 20 | □ |
| 7 | Check first feature coordinates: grep -A3 "coordinates" | Coordinates near 37.49, -113.3x (New Harmony UT range) | □ |
| 8 | Verify CORS header: curl -I http://localhost:8765/convoy.geojson ¦ grep Access-Control | Access-Control-Allow-Origin: * present | □ |
| 9 | Verify Content-Type header: curl -I http://localhost:8765/convoy.geojson ¦ grep Content-Type | Content-Type: application/geo+json | □ |
| 10 | Verify Cache-Control: curl -I http://localhost:8765/convoy.geojson ¦ grep Cache-Control | Cache-Control: no-cache, no-store, must-revalidate | □ |

| PASS if: 20 features returned. Type=FeatureCollection. Coords in New Harmony range. CORS and no-cache headers present. | On FAIL: Check server terminal for error output. Verify geojson_server.py node coordinates are set to New Harmony UT. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-03 Status Endpoint Returns Valid JSON Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Run: curl -s http://localhost:8765/status ¦ python3 -m json.tool | Formatted JSON output | □ |
| 2 | Verify "status": "live" | status field = "live" | □ |
| 3 | Verify "units": 20 | units = 20 | □ |
| 4 | Verify "active" count > 0 | active field is a positive integer | □ |
| 5 | Verify "nodes" array present with 20 entries | nodes array length = 20 | □ |
| 6 | Check a node entry for required fields: callsign, lat, lon, speed_mph, battery, status, last_seen_ago | All fields present in first node entry | □ |
| 7 | Run curl again after 10 seconds. Check rev field incremented | "rev" value is higher than first call | □ |

| PASS if: Valid JSON. 20 units. Required fields present. Rev increments on repeated calls. | On FAIL: Check server code for /status handler. Verify nodes array is populated. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-04 Phone Can Reach Server on LAN Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-01 passed. Phone connected to laptop hotspot or same WiFi. LAN IP noted from TC-A-01. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | On laptop: confirm hotspot is active and phone is connected | Phone shows connected to hotspot network | □ |
| 2 | On phone: open Safari or Chrome | Browser opens | □ |
| 3 | Navigate to: http://[LAN-IP]:8765/status | JSON response loads in browser | □ |
| 4 | Verify JSON shows status: live, units: 20 | Data visible in browser — not a connection error | □ |
| 5 | Navigate to: http://[LAN-IP]:8765/convoy.geojson | GeoJSON loads or browser prompts to download — no timeout or refused connection | □ |
| 6 | Check server terminal for incoming request log entries | Server logs show GET requests from phone IP | □ |

| PASS if: Phone browser can load /status and /convoy.geojson from LAN IP. No timeout or connection refused. | On FAIL: Check firewall rules on laptop. macOS: System Settings → Firewall → ensure port 8765 allowed. Check hotspot subnet routing. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-05 Gaia GPS Imports GeoJSON Custom Map Source Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-04 passed. Gaia GPS installed and logged in. LAN IP confirmed reachable from phone. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open Gaia GPS on phone | Gaia map loads | □ |
| 2 | Tap Menu → Account (or Settings) → Custom Map Sources | Custom Map Sources screen opens | □ |
| 3 | Tap Add New Source or + button | URL entry dialog appears | □ |
| 4 | Enter URL: http://[LAN-IP]:8765/convoy.geojson | URL accepted. Source named. | □ |
| 5 | Set name: Convoy Tracker. Save. | Source appears in list | □ |
| 6 | Return to map. Open Layers panel. | Convoy Tracker layer visible in list | □ |
| 7 | Enable the Convoy Tracker layer | Layer toggles on — no error shown | □ |
| 8 | Check server terminal for GET request from Gaia | Server log shows Gaia fetching /convoy.geojson | □ |

| PASS if: Custom map source saved. Layer enabled. Server receives fetch request from Gaia. | On FAIL: Verify URL format. Check Gaia GPS version supports Custom Map Sources. Try onX as alternative. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-06 Convoy Markers Appear on Terrain at New Harmony UT Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-05 passed. Gaia map open with Convoy Tracker layer enabled. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | In Gaia GPS, search for or navigate to: New Harmony, Utah | Map centers on New Harmony UT area | □ |
| 2 | Zoom to level where individual roads are visible | New Harmony Rd visible on map | □ |
| 3 | Wait up to 60 seconds for Gaia to refresh the layer | Markers begin appearing on map | □ |
| 4 | Count visible markers | Approximately 20 markers visible | □ |
| 5 | Verify marker positions follow New Harmony Rd NW toward Pine Valley Mountain | Marker string runs NW along road corridor — not random | □ |
| 6 | Verify lead marker (LEAD-1) is furthest NW near 37.501, -113.330 | Lead marker in correct geographic position | □ |
| 7 | Verify tail marker (TAIL-20) is furthest SE near 37.475, -113.298 | Tail marker near I-15 corridor | □ |
| 8 | Zoom in on any marker — confirm it snaps to road or road proximity | Markers on or near road — not in water or impossible terrain | □ |

| PASS if: 20+ markers visible. Markers positioned along New Harmony Rd NW corridor. Lead/tail in correct relative positions. | On FAIL: CRITICAL GATE FAILURE. Document: were any markers visible? Wrong location? No markers at all? Proceed to TC-B. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-07 Marker Tap Shows Correct Telemetry Popup Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-06 passed. Markers visible on Gaia map. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Tap the LEAD-1 marker (northwesternmost marker) | Popup or info card appears | □ |
| 2 | Verify popup shows: callsign (LEAD-1) | Callsign field present and correct | □ |
| 3 | Verify popup shows: role (Lead) | Role = Lead | □ |
| 4 | Verify popup shows: speed in mph | Speed field present with numeric value ~14 mph | □ |
| 5 | Verify popup shows: battery percentage | Battery field present with value ~91% | □ |
| 6 | Verify popup shows: status (ACTIVE) | Status = ACTIVE | □ |
| 7 | Verify popup shows: last seen age (e.g. 3s ago, 12s ago) | Age field present and updating | □ |
| 8 | Tap HOTEL-10 (position 10 — My Cart, marked with star symbol) | Popup shows is_my_cart = true or Role = My Cart | □ |

| PASS if: Popup visible on tap. All required fields (callsign, role, speed, battery, status, last_seen_ago) present with plausible values. | On FAIL: Check GeoJSON properties schema. Verify 'description' field is properly formatted in nodes_to_geojson(). | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-08 Markers Update — Position and Status Refresh Suite A — Gate 1 ¦ CRITICAL

| Prerequisites | TC-A-07 passed. Gaia displaying live markers. Note positions of 2-3 markers. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Note current position of LEAD-1 marker on screen | Position recorded (screenshot or mental note) | □ |
| 2 | Wait 60-90 seconds (Gaia refresh cycle) | Gaia re-fetches the GeoJSON source | □ |
| 3 | Observe LEAD-1 position after refresh | Marker has moved slightly NW — position updated | □ |
| 4 | Check server terminal — confirm additional GET requests logged | Multiple fetch requests visible in server log | □ |
| 5 | Check 'rev' value in /status via phone browser — confirm it has incremented | Rev number is higher than initial value | □ |
| 6 | Verify no markers disappeared or duplicated during refresh | Still ~20 markers, no duplicates | □ |

| PASS if: Marker positions change between Gaia refresh cycles. Server logs show repeated fetches. Rev increments. | On FAIL: Check Gaia refresh interval settings. Manually force a layer refresh if available. Check simulation_loop() tick rate. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-09 LOST Node Marker Turns Red Suite A — Gate 1 ¦ HIGH

| Prerequisites | TC-A-08 passed. geojson_server.py running with simulation. Note marker colors. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Observe current marker colors on Gaia — note all are colored (not red) | All markers show node colors (blue, orange, green, etc.) | □ |
| 2 | Wait 3-4 minutes without restarting server (simulation dropout will trigger stale nodes) | Server continues running | □ |
| 3 | Trigger a manual stale: in second terminal, note a specific node callsign | Target node identified | □ |
| 4 | After Gaia refresh cycle, observe marker colors | At least 1-2 markers have turned red (#FF453A) | □ |
| 5 | Tap a red marker. Verify popup shows status: LOST | Status field = LOST in popup | □ |
| 6 | Tap a non-red marker. Verify status: ACTIVE | Active nodes still show status = ACTIVE | □ |
| 7 | Wait for simulation recovery (20% per tick). Observe red marker eventually turn back to original color | Node recovers — marker color reverts | □ |

| PASS if: Stale nodes show red marker-color in Gaia. Popup shows LOST. Recovery restores original color. | On FAIL: Check STALE_SECONDS constant in geojson_server.py (120s). Check marker-color property in nodes_to_geojson(). | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-A-10 onX Custom Layer Import (Secondary Validation) Suite A — Gate 1 ¦ HIGH

| Prerequisites | TC-A-08 passed. onX Hunt or onX Offroad installed on phone. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open onX Hunt or onX Offroad on phone | onX map opens | □ |
| 2 | Navigate to Custom Layers or Map Layers menu | Layer management screen visible | □ |
| 3 | Add new custom GeoJSON layer or map source | URL input available | □ |
| 4 | Enter URL: http://[LAN-IP]:8765/convoy.geojson | URL accepted | □ |
| 5 | Navigate to New Harmony, Utah area on onX map | Map shows New Harmony UT terrain | □ |
| 6 | Enable the convoy layer — wait for refresh | Markers appear on onX map in NW corridor | □ |
| 7 | Tap a marker — verify popup shows telemetry fields | Name, description, role, speed, battery visible | □ |

| PASS if: Markers visible in onX at correct New Harmony UT locations. Popup shows telemetry. | On FAIL: onX may require specific subscription tier for custom GeoJSON. Document tier requirement. Gaia remains primary. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite B — Gate 2: Offline Map Sharing (Path B1)

ENTRY CRITERIA: TC-A-06 FAILED — GeoJSON markers did not appear in Gaia/onX.

GATE DECISION: If TC-B-01 through TC-B-04 PASS → Path B1 viable. Design tile-backed standalone app.

If TC-B-01 through TC-B-04 FAIL → Skip to Suite TC-C (Path B2 — Tracks Only).

TC-B-01 Gaia GPS Can Export Offline Map Package Suite B — Gate 2 ¦ HIGH

| Prerequisites | Gaia GPS installed. New Harmony UT area downloaded in Gaia. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open Gaia GPS. Navigate to New Harmony UT area (37.4794, -113.3097) | Area visible on Gaia map | □ |
| 2 | Download offline maps for 10-mile radius at zoom levels 10-15 | Download completes. Tile cache available. | □ |
| 3 | Navigate to Gaia Settings → Manage Downloads or Map Cache | Downloaded map packages visible | □ |
| 4 | Attempt to export or share the map package to Files app or a shared location | Share/export option exists | □ |
| 5 | Export produces a file — note file format (.mbtiles, .gpkg, .zip, or other) | File appears in Files app or export destination | □ |
| 6 | Note file size | File size recorded | □ |

| PASS if: Gaia produces an exportable map file in a known format. File is accessible outside Gaia sandbox. | On FAIL: Gaia may not expose map cache for export. Document limitation. This may immediately trigger Path B2. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-B-02 Tile File Is Valid and Readable Suite B — Gate 2 ¦ HIGH

| Prerequisites | TC-B-01 passed. Map file exported. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Transfer file to laptop (AirDrop, USB, cloud) | File accessible on laptop | □ |
| 2 | If .mbtiles: run: sqlite3 [file.mbtiles] 'SELECT COUNT(*) FROM tiles;' | Tile count returned (thousands expected) | □ |
| 3 | If .mbtiles: run: sqlite3 [file.mbtiles] 'SELECT zoom_level, COUNT(*) FROM tiles GROUP BY zoom_level;' | Zoom levels 10-15 populated | □ |
| 4 | Attempt to open in QGIS or TileMill for visual verification | New Harmony UT terrain visible at target zoom levels | □ |
| 5 | Confirm bounding box includes 37.4-37.6 lat, -113.4 to -113.2 lon | Coverage confirmed over target area | □ |

| PASS if: File is valid SQLite/MBTiles. Tiles present for zoom 10-15. Coverage includes New Harmony UT. | On FAIL: File may be in non-standard format. Check if MapLibre GL or Leaflet can consume the format. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-B-03 Tile File Can Be Shared to App Sandbox on iOS/Android Suite B — Gate 2 ¦ HIGH

| Prerequisites | TC-B-02 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | On iOS: attempt to share .mbtiles file to a test app via Share Sheet → Copy to [App] | Share sheet offers target app as destination | □ |
| 2 | On Android: place .mbtiles in app-accessible storage (Downloads, or explicit path) | File accessible from target app | □ |
| 3 | Verify file is not corrupted after transfer (check file size matches source) | File size matches within 1% | □ |
| 4 | Attempt to open tile file from within a MapLibre-capable test app or web view | Tiles render — terrain visible | □ |

| PASS if: Tile file successfully transferred to target app and renders terrain. | On FAIL: iOS sandbox restrictions may prevent this. Document the platform-specific blocker. Proceed to TC-C. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-B-04 Convoy GeoJSON Overlays Correctly on Tile Background Suite B — Gate 2 ¦ HIGH

| Prerequisites | TC-B-03 passed. Tiles loading in test environment. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Start geojson_server.py on laptop | Server running, LAN IP printed | □ |
| 2 | In test app with tile background: add GeoJSON data source pointing to /convoy.geojson | GeoJSON source added | □ |
| 3 | Confirm convoy markers appear on top of tile terrain | Markers visible on terrain background | □ |
| 4 | Verify marker positions align correctly with road geometry in tiles | Markers on or near road — not offset | □ |
| 5 | Verify zoom and pan work — tiles and markers move together | Coordinate registration maintained at all zoom levels | □ |

| PASS if: GeoJSON markers render correctly on tile background. Coordinate systems aligned. Zoom/pan functional. | On FAIL: Coordinate projection mismatch (WGS84 vs Web Mercator). Check tile CRS vs GeoJSON CRS. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-B-05 Path B1 UX Acceptability Assessment Suite B — Gate 2 ¦ MEDIUM

| Prerequisites | TC-B-04 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Evaluate two-app workflow: open standalone convoy tracker, note context switch to Gaia required for navigation | Context switch mechanics understood | □ |
| 2 | Time a context switch from convoy tracker to Gaia and back | Round-trip time recorded (target: under 5 seconds) | □ |
| 3 | Assess: can driver safely alt-tab while convoy is moving? | UX acceptability noted | □ |
| 4 | Assess: does tile background provide sufficient situational awareness vs no map? | Yes/No recorded with notes | □ |

| PASS if: Team agrees two-app workflow is acceptable for operational use. | On FAIL: If not acceptable, proceed to TC-C and recommend Path B2 directly. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-B-06 Offline Tile Coverage at Field Location Suite B — Gate 2 ¦ MEDIUM

| Prerequisites | TC-B-04 passed. Target field area defined. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Confirm tiles cover entire 3-mile convoy route from 37.475 to 37.502 lat | Both ends of route have tile coverage | □ |
| 2 | Verify tiles remain available with no internet connection (enable airplane mode on phone) | Tiles continue rendering with no connectivity | □ |
| 3 | Pan to areas just outside downloaded region — confirm graceful degradation | App shows empty/gray areas rather than crashing | □ |

| PASS if: Full convoy route covered by tiles at zoom 12-14. Tiles render offline. | On FAIL: Expand tile download area in Gaia before re-testing. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite C — Gate 3: Tracks Only (Path B2)

ENTRY CRITERIA: TC-B tests FAILED or Path B1 rejected as operationally unacceptable.

NOTE: This path is always viable. These tests confirm the baseline convoy_tracker_v4.jsx functions correctly.

TC-C-01 Standalone TacticalMap Renders Correctly Suite C — Gate 3 ¦ MEDIUM

| Prerequisites | convoy_tracker_v4.jsx accessible in browser. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open convoy_tracker_v4.jsx artifact in browser | Tactical map view loads without errors | □ |
| 2 | Verify 20 node markers visible on coordinate grid | All 20 nodes represented | □ |
| 3 | Verify lead marker (triangle up) at NW position | LEAD-1 at top of map | □ |
| 4 | Verify tail marker (triangle down) at SE position | TAIL-20 at bottom of map | □ |
| 5 | Verify convoy spine line connects lead to tail through all nodes | Spine line visible | □ |
| 6 | Verify stale nodes show red blinking dart markers | Stale nodes visually distinct | □ |

| PASS if: All 20 nodes visible. Lead/tail correct. Spine line present. Stale nodes highlighted. | On FAIL: Check convoy_tracker_v4.jsx for rendering errors in browser console. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-C-02 Grid HUD and Telemetry Panel Operational Suite C — Gate 3 ¦ MEDIUM

| Prerequisites | TC-C-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Verify grid HUD panel shows in upper-left area | Grid HUD visible with node list | □ |
| 2 | Verify CONVOY / MY CART toggle switches grid content | Toggle changes displayed data | □ |
| 3 | Tap a node row in grid — verify detail panel appears with coords, speed, battery | Detail panel shows | □ |
| 4 | Verify CONTACT LOST banner appears when stale nodes present | Alert banner visible | □ |
| 5 | Verify server status badge shows correct state | Badge reflects server up/down | □ |

| PASS if: All HUD panels functional. Toggles work. Detail panel shows. Alerts fire. | On FAIL: Check browser console for JS errors. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-C-03 Coordinate Grid Accurate for New Harmony UT Suite C — Gate 3 ¦ MEDIUM

| Prerequisites | TC-C-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Verify node coordinates displayed in grid match New Harmony UT range (37.47-37.50 lat, -113.28 to -113.33 lon) | Coordinates in expected range | □ |
| 2 | Cross-reference LEAD-1 displayed coordinates against known value (37.5012, -113.3298) | Match within 4 decimal places | □ |
| 3 | Verify convoy span calculation reflects ~3 mile spread | Span shown as ~3.0 mi or ~15840 ft | □ |

| PASS if: Coordinates accurate. Span calculation correct for New Harmony UT layout. | On FAIL: Check makeNodes() in convoy_tracker_v4.jsx for coordinate values. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-C-04 Path B2 Operator Briefing Assessment Suite C — Gate 3 ¦ LOW

| Prerequisites | TC-C-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Brief a non-developer operator on how to interpret the coordinate grid without terrain context | Operator understands spacing and direction | □ |
| 2 | Ask operator to identify which vehicle is lead, which is tail, and which is theirs (HOTEL-10) | Operator correctly identifies all three within 60 seconds | □ |
| 3 | Simulate a LOST node — ask operator to identify which vehicle lost contact | Operator identifies lost vehicle from blinking dart and banner | □ |
| 4 | Ask operator to assess whether Path B2 is acceptable for their operational use case | Acceptability noted | □ |

| PASS if: Operator can use system effectively without terrain background. Acceptability documented. | On FAIL: Path B2 is always viable technically. Operational acceptability is a human factors decision. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite D — Backend Server

ENTRY CRITERIA: meshtastic_backend.py available. Python 3.9+ installed.

NOTE: Suite D is common to all paths. Run concurrently with or before Suite A.

TC-D-01 Backend Starts and Serves All Endpoints Suite D — Backend ¦ CRITICAL

| Prerequisites | meshtastic_backend.py in working directory. No Meshtastic hardware required (test server-only mode). |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Run: python3 meshtastic_backend.py --port 8765 (or equivalent no-device mode) | Server starts. LAN IP and URLs printed. | □ |
| 2 | Verify startup output shows GeoJSON URL, KML URL, status URL | All three URLs printed | □ |
| 3 | curl http://localhost:8765/convoy.geojson — expect empty FeatureCollection or minimal response | Valid JSON returned (no nodes yet) | □ |
| 4 | curl http://localhost:8765/status — verify JSON structure | JSON with status, rev, units fields | □ |
| 5 | curl http://localhost:8765/convoy.kml — verify XML returned | XML with KML root element | □ |
| 6 | curl http://localhost:8765/networklink.kml — verify NetworkLink wrapper | NetworkLink XML returned | □ |
| 7 | curl -X OPTIONS http://localhost:8765/convoy.geojson -I — check CORS preflight | 200 OK with Access-Control headers | □ |

| PASS if: All 5 endpoints respond. CORS headers present. No server errors. | On FAIL: Check Python imports. Verify meshtastic library not required for server-only mode. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-02 CORS Headers Present on All Responses Suite D — Backend ¦ CRITICAL

| Prerequisites | TC-D-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | curl -I http://localhost:8765/convoy.geojson ¦ grep -i access-control | Access-Control-Allow-Origin: * | □ |
| 2 | curl -I http://localhost:8765/status ¦ grep -i access-control | Access-Control-Allow-Origin: * | □ |
| 3 | curl -I http://localhost:8765/convoy.kml ¦ grep -i access-control | Access-Control-Allow-Origin: * | □ |
| 4 | curl -I http://localhost:8765/networklink.kml ¦ grep -i access-control | Access-Control-Allow-Origin: * | □ |
| 5 | OPTIONS preflight: curl -X OPTIONS -I http://localhost:8765/convoy.geojson | 200 OK. Access-Control-Allow-Methods: GET, OPTIONS | □ |

| PASS if: All endpoints return CORS headers. OPTIONS returns 200. | On FAIL: Check _send_cors() is called in all response branches of KMLRequestHandler. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-03 GeoJSON Properties Schema Complete Suite D — Backend ¦ CRITICAL

| Prerequisites | TC-D-01 passed. Live Meshtastic data available OR manually inject test node. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Fetch /convoy.geojson and examine first feature's properties | Properties object present | □ |
| 2 | Verify: name, description, marker-color, marker-symbol, marker-size | All 5 simplestyle fields present | □ |
| 3 | Verify: callsign, node_id, role, cot_type | All 4 identity fields present | □ |
| 4 | Verify: speed_mph, battery_pct, snr_db, altitude_m, dist_to_lead_ft | All 5 telemetry fields present | □ |
| 5 | Verify: status (ACTIVE/LOST), age_s, last_seen_ago, timestamp_utc | All 4 status/time fields present | □ |
| 6 | Verify: is_lead, is_tail, is_my_cart booleans | All 3 role booleans present | □ |

| PASS if: All 22 required properties present in every feature. No null required fields. | On FAIL: Check build_geojson() in ConvoyManager. Add any missing property mappings. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-04 Stale Node Stays in GeoJSON as LOST Suite D — Backend ¦ CRITICAL

| Prerequisites | TC-D-01 passed. Backend running with test nodes. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Inject a test node with a last_seen timestamp > 120 seconds ago | Node in carts dict | □ |
| 2 | Fetch /convoy.geojson — find the stale node's feature | Feature present in response (not missing) | □ |
| 3 | Verify stale node's status property = LOST | "status": "LOST" | □ |
| 4 | Verify stale node's marker-color = #FF453A | "marker-color": "#FF453A" | □ |
| 5 | Verify stale node is NOT removed at 120s mark — only after 300s (5 min) | Node present in /status nodes array at 2 min mark | □ |
| 6 | Wait for node to exceed 300s age. Verify it is now absent from /status | Node pruned after 5 min | □ |

| PASS if: Stale node remains in GeoJSON as LOST until 300s. Prune only after 5 min. | On FAIL: Check PRUNE_AFTER constant and _update_loop() in ConvoyManager. Verify is_stale() threshold. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-05 Lead/Tail Assignment — Heading-Based Sort Suite D — Backend ¦ HIGH

| Prerequisites | TC-D-01 passed. Multiple test nodes with track history. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Inject 5 nodes moving in a NW direction — add 3+ position history points each | Nodes have track_history populated | □ |
| 2 | Call _update_lead_tail() — inspect results | Lead assigned to NW-most node. Tail to SE-most. | □ |
| 3 | Verify sort is NOT based on raw lat+lon sum (old bug) | Correct result for E-W route: westernmost = lead | □ |
| 4 | Test E-W route: add nodes moving due west. Verify westernmost = lead | Lead correctly assigned for E-W convoy | □ |
| 5 | Test with nodes having no history: verify lat-sort fallback works | Northernmost node = lead when no history | □ |

| PASS if: Lead/tail correct for NW, N-S, and E-W routes. Fallback to lat-sort when no history. | On FAIL: Check _update_lead_tail() sort_key function. Verify dot-product heading projection. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-06 Exponential Backoff Reconnect on Device Disconnect Suite D — Backend ¦ HIGH

| Prerequisites | TC-D-01 passed. Meshtastic hardware OR mock interface. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Connect backend to Meshtastic device. Confirm connection in logs. | Connection logged | □ |
| 2 | Physically disconnect Meshtastic device (unplug USB) | Backend logs 'connection lost' | □ |
| 3 | Monitor reconnect attempt timing: note time of first attempt | Attempt ~1s after disconnect | □ |
| 4 | Second attempt — note delay | ~2s delay (doubling) | □ |
| 5 | Continue monitoring — verify delays double: 1, 2, 4, 8, 16, 30, 30... | Backoff sequence confirms to doubling pattern, caps at 60s | □ |
| 6 | Reconnect device. Verify backend detects and logs successful reconnect | 'reconnected successfully' in logs | □ |
| 7 | Verify backoff resets to 1s after successful reconnect | Next test disconnect shows 1s first attempt | □ |

| PASS if: Backoff sequence follows 1→2→4→8→16→30→30... pattern. Caps at 60s. Resets on reconnect. | On FAIL: Check _watchdog() in MeshtasticListener. Verify backoff and MAX_BACKOFF constants. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-07 COT Type Parsed and Stored Suite D — Backend ¦ HIGH

| Prerequisites | TC-D-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Send a COT packet with type='a-f-G-U-C' (friendly ground) | Packet processed | □ |
| 2 | Fetch /convoy.geojson — find node. Verify cot_type = a-f-G-U-C | "cot_type": "a-f-G-U-C" | □ |
| 3 | Send COT packet with type='a-h-G-U-C' (hostile ground) | Packet processed | □ |
| 4 | Verify cot_type updated to a-h-G-U-C in next /convoy.geojson response | cot_type reflects hostile type | □ |
| 5 | Verify cot_type persists in /status nodes array | cot_type in /status matches /convoy.geojson | □ |

| PASS if: COT type correctly parsed, stored in node state, and reflected in both GeoJSON and status endpoints. | On FAIL: Check COTParser.parse() and ConvoyCart.update_from_cot() cot_type handling. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-D-08 Battery Updated via TELEMETRY_APP Packet Suite D — Backend ¦ MEDIUM

| Prerequisites | TC-D-01 passed. Meshtastic device sending telemetry. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Send a TELEMETRY_APP packet with deviceMetrics.batteryLevel = 42 | Packet received by backend | □ |
| 2 | Fetch /convoy.geojson — find affected node | Feature present | □ |
| 3 | Verify battery_pct = 42 in properties | "battery_pct": 42 | □ |
| 4 | Verify /status also shows updated battery value for that node | battery = 42 in /status nodes array | □ |
| 5 | Send POSITION_APP packet without battery — verify battery retains last telemetry value | battery_pct does not revert to null | □ |

| PASS if: Battery level from TELEMETRY_APP stored and reflected in GeoJSON and status. Persists across position updates. | On FAIL: Check _handle_telemetry() in MeshtasticListener and battery update path in _on_position(). | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite E — Overlay HUD (Path A)

ENTRY CRITERIA: TC-A-08 passed. geojson_server.py or meshtastic_backend.py running. Phone browser available.

TC-E-01 Overlay Renders Transparent Over Gaia Suite E — Overlay HUD ¦ CRITICAL

| Prerequisites | Gaia GPS open with convoy layer. convoy_overlay.jsx accessible in browser on same phone. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open convoy_overlay.jsx in phone browser (Safari or Chrome) | Overlay loads without error | □ |
| 2 | Use split-screen or floating window to position overlay over Gaia (or alt-tab rapidly) | Both apps visible | □ |
| 3 | Verify overlay background is transparent — Gaia terrain visible behind frosted glass panels | Map shows through empty overlay areas | □ |
| 4 | Verify frosted glass HUD panel does not fully obscure the map | Map readable around HUD panels | □ |
| 5 | Pinch to zoom in Gaia while overlay is visible — verify gesture passes through | Gaia responds to zoom gesture | □ |
| 6 | Pan Gaia map while overlay is visible — verify pan gesture passes through transparent areas | Map pans correctly | □ |

| PASS if: Transparent background confirmed. Touch passthrough works for zoom and pan. Map readable behind overlay. | On FAIL: Check pointer-events: none on root overlay element. Check backdrop-filter support in target browser. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-02 Grid HUD Shows All 20 Nodes Suite E — Overlay HUD ¦ CRITICAL

| Prerequisites | TC-E-01 passed. Overlay running with simulated or live data. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Verify Grid HUD panel is visible in upper-left of overlay | Frosted glass panel present | □ |
| 2 | Count node rows in grid | 20 rows visible (or scroll available if list exceeds panel height) | □ |
| 3 | Verify each row shows: callsign, age (e.g. 3s), battery bar, speed, signal bars | All 5 data columns present per row | □ |
| 4 | Verify lead node row styled distinctly (e.g. #00D4FF accent) | LEAD-1 visually distinct | □ |
| 5 | Verify stale node rows show red tint and ⚠ suffix on callsign | Stale rows styled red | □ |
| 6 | Tap a grid row — verify detail panel slides up | Detail panel shows node coords, distances | □ |
| 7 | Tap × on detail panel — verify it closes | Panel dismisses | □ |

| PASS if: All nodes listed. Data columns correct. Stale styling visible. Detail panel opens and closes. | On FAIL: Check GridHUD component. Verify node data schema matches overlay state variables. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-03 CONVOY / MY CART Toggle Switches View Suite E — Overlay HUD ¦ HIGH

| Prerequisites | TC-E-02 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Verify default view shows CONVOY bottom strip | Convoy stats visible at bottom | □ |
| 2 | Tap CONVOY button in HUD header to switch to MY CART | Bottom strip changes to cart telemetry | □ |
| 3 | MY CART strip shows: callsign HOTEL-10, lat/lon, speed, battery, SNR, altitude | All telemetry fields present | □ |
| 4 | MY CART strip shows proximity cards: AHEAD, BEHIND, LEAD, TAIL | 4 proximity cards visible | □ |
| 5 | Proximity cards show callsign and distance for nearest units | e.g. GOLF-9 0.17 mi BEHIND | □ |
| 6 | Tap MY CART button to switch back to CONVOY view | CONVOY strip returns | □ |
| 7 | CONVOY strip shows: UNITS count, SPAN distance, ACTIVE count, LOST count | All 4 convoy stats present | □ |

| PASS if: Toggle switches views correctly. MY CART shows correct telemetry and proximity. CONVOY shows correct stats. | On FAIL: Check view state variable and CartStrip/ConvoyStrip components. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-04 CONTACT LOST Banner Fires and Clears Suite E — Overlay HUD ¦ CRITICAL

| Prerequisites | TC-E-02 passed. Overlay running with simulated data. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Note: simulation has 8% per-tick dropout. Wait 2+ minutes for stale nodes. | Stale nodes appear in grid with red tint | □ |
| 2 | Verify CONTACT LOST alert banner appears below grid HUD | Red alert panel visible with blinking ! icon | □ |
| 3 | Verify banner lists callsigns of lost nodes and staleness duration | e.g. 'OSCAR-17 3m 24s' | □ |
| 4 | Verify banner shows count: ACTIVE X / LOST Y | Counts match grid stale row count | □ |
| 5 | Wait for a lost node to recover (simulation 20% per tick) | Node callsign removed from banner | □ |
| 6 | If all nodes recover, banner should disappear | Banner hides when lost.length = 0 | □ |

| PASS if: Banner appears when any node stale. Lists correct nodes. Clears on recovery. | On FAIL: Check LostBanner component and lost node filtering logic in App component. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-05 Server Status Badge States Suite E — Overlay HUD ¦ HIGH

| Prerequisites | TC-E-01 passed. Backend server accessible. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Start server. Verify badge shows green beacon with 'SRV 8765' and 'REV n' | Live state confirmed | □ |
| 2 | Kill server process. Wait 10 seconds. | Badge transitions to down state | □ |
| 3 | Verify badge shows red dot with 'SRV DOWN' text | Down state displayed | □ |
| 4 | Verify 'geojson offline' subtext appears in down state | Offline subtext visible | □ |
| 5 | Restart server. Wait for reconnect. | Badge returns to live state | □ |
| 6 | Verify 'REV' count is higher than before kill | Rev incremented on reconnect | □ |

| PASS if: Badge correctly shows live/down/restarting states. Transitions on server kill and restart. | On FAIL: Check SrvBadge component and server polling logic. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-06 Grid HUD Collapses to Pill Button Suite E — Overlay HUD ¦ MEDIUM

| Prerequisites | TC-E-02 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Locate × close button in grid HUD header | Close button visible | □ |
| 2 | Tap × button | Grid HUD collapses | □ |
| 3 | Verify collapsed state shows pill button: ☰ [count] ⚠N | Pill shows unit count and warning count | □ |
| 4 | Verify pill button positioned so it minimally obstructs map | Small footprint confirmed | □ |
| 5 | Tap pill button | Grid HUD expands back | □ |
| 6 | Verify grid content unchanged after collapse/expand cycle | Data preserved | □ |

| PASS if: HUD collapses to pill. Pill shows correct counts. Re-expands correctly. | On FAIL: Check showGrid state and collapse/expand logic in GridHUD. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-07 Live Data Polling from /status Endpoint Suite E — Overlay HUD ¦ CRITICAL

| Prerequisites | meshtastic_backend.py running with live Meshtastic data OR geojson_server.py as substitute. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Open overlay. Verify server badge shows live. | Live badge confirmed | □ |
| 2 | Note a node's speed value in the grid (e.g. SCOUT-2 = 13.8 mph) | Initial value recorded | □ |
| 3 | Wait 10 seconds (2 polling cycles) | Grid refreshes | □ |
| 4 | Verify SCOUT-2 speed has changed slightly (simulation jitter) | Speed value updated | □ |
| 5 | Note node age values — verify they increment in real time | Age ticking up correctly | □ |
| 6 | Kill backend. Verify overlay retains last known values (does not zero out) | Stale data persists — no blank fields | □ |
| 7 | Verify server badge transitions to SRV DOWN within 10 seconds of kill | Down state shown | □ |

| PASS if: Overlay polls /status every 5s. Data updates visible. Stale retention on disconnect. Badge reflects server state. | On FAIL: Check useEffect polling loop in App component. Verify /status response maps correctly to overlay state. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-08 Overlay Legible in Bright Outdoor Light Suite E — Overlay HUD ¦ HIGH

| Prerequisites | Phone with overlay loaded. Outdoor or simulated bright light environment. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Take phone outside or hold under bright indoor light | Display visible | □ |
| 2 | Read callsign, speed, and battery values from grid HUD | Text legible without strain | □ |
| 3 | Read convoy stats (UNITS, SPAN, ACTIVE) from bottom strip | Legible | □ |
| 4 | Identify a red stale node row vs active rows | Color distinction visible in bright light | □ |
| 5 | Locate and tap the CONVOY/MY CART toggle button | Button target reachable with gloves if applicable | □ |
| 6 | Note any readability issues: font size, contrast, panel opacity | Issues documented for adjustment | □ |

| PASS if: All critical information legible in bright conditions. Color distinctions visible. Touch targets reachable. | On FAIL: Increase font size, boost contrast, or increase panel opacity for outdoor readability. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-E-09 Overlay Landscape Orientation Suite E — Overlay HUD ¦ MEDIUM

| Prerequisites | TC-E-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Rotate phone to landscape orientation | Overlay reflows or remains functional | □ |
| 2 | Verify grid HUD still accessible and readable in landscape | No overflow or clipping | □ |
| 3 | Verify bottom strip remains visible and not cut off | Strip fully visible | □ |
| 4 | Rotate back to portrait | Layout returns correctly | □ |

| PASS if: Overlay functional in both orientations. No critical UI clipping. | On FAIL: Add CSS orientation media queries or constrain to portrait mode for v1 if needed. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite F — Live Meshtastic Integration

ENTRY CRITERIA: TC-D-01 passed. Minimum 2 Meshtastic devices with GPS fix confirmed.

TC-F-01 Backend Connects to Meshtastic Device Suite F — Meshtastic ¦ CRITICAL

| Prerequisites | Meshtastic device connected via USB to laptop. meshtastic_backend.py installed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Connect Meshtastic device via USB. Identify serial port (e.g. /dev/ttyUSB0 or /dev/cu.usbserial-*) | Port identified | □ |
| 2 | Run: python3 meshtastic_backend.py --port /dev/[port] | No connection error | □ |
| 3 | Verify log: 'Meshtastic connected via serial' | Connection confirmed in logs | □ |
| 4 | Verify log: 'Seeded N node names from radio database' | Node registry populated | □ |
| 5 | Fetch /status — verify units count matches known mesh size | Unit count reasonable | □ |

| PASS if: Backend connects. Node names seeded. /status reports units. | On FAIL: Check USB permissions. macOS: may need to install CH340 driver. Try --tcp-host for WiFi bridge. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-02 POSITION_APP Packets Update Node State Suite F — Meshtastic ¦ CRITICAL

| Prerequisites | TC-F-01 passed. GPS fix confirmed on at least one remote node. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Verify remote node has GPS fix (LED or Meshtastic app confirmation) | GPS fix confirmed | □ |
| 2 | Watch backend logs for POSITION_APP messages from remote node | Position packets logged | □ |
| 3 | Fetch /convoy.geojson — find remote node feature | Feature present | □ |
| 4 | Verify coordinates are current GPS position (not 0,0 or stale) | Lat/lon plausible for location | □ |
| 5 | Move the remote node 50+ feet. Wait 30s. Fetch /convoy.geojson again. | Coordinates updated to new position | □ |
| 6 | Verify speed value is non-zero while node is moving | speed_mph > 0 during movement | □ |

| PASS if: Remote GPS positions appear in GeoJSON. Positions update when node moves. Speed non-zero during movement. | On FAIL: Check _handle_position() in MeshtasticListener. Verify latitudeI/1e7 scaling if using integer lat/lon. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-03 COT Packets Parsed and Applied Suite F — Meshtastic ¦ HIGH

| Prerequisites | TC-F-01 passed. Second device configured to send COT messages. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Send a COT XML message via TEXT_MESSAGE_APP from remote device | Backend logs COT receive | □ |
| 2 | Verify COTParser.is_cot() correctly identifies the payload | is_cot returns True | □ |
| 3 | Verify parsed callsign appears in /convoy.geojson for that node | callsign updated to COT value | □ |
| 4 | Verify cot_type updated in GeoJSON properties | cot_type = value from COT packet | □ |
| 5 | Send COT with position data (<point> element) — verify GeoJSON position updated | Position from COT applied | □ |

| PASS if: COT callsigns and types parsed and reflected in GeoJSON within one update cycle. | On FAIL: Check COTParser.parse() XML extraction. Verify _handle_cot() callback chain. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-04 TELEMETRY_APP Battery Updates Applied Suite F — Meshtastic ¦ HIGH

| Prerequisites | TC-F-01 passed. Meshtastic device sending telemetry. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Note current battery_pct in /convoy.geojson for local node | Initial battery value recorded | □ |
| 2 | Wait for TELEMETRY_APP packet (Meshtastic sends every ~60s) | Telemetry packet logged | □ |
| 3 | Fetch /convoy.geojson — verify battery_pct updated | battery_pct reflects current device battery | □ |
| 4 | Cross-reference against Meshtastic app battery display | Values match within 5% | □ |

| PASS if: Battery from TELEMETRY_APP reflected in GeoJSON. Matches Meshtastic app value. | On FAIL: Check _handle_telemetry() battery update path. Verify TELEMETRY_APP portnum handling. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-05 Node Recovery After Loss of Contact Suite F — Meshtastic ¦ HIGH

| Prerequisites | TC-F-02 passed. Two active nodes. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Move remote node outside LoRa range (or power off briefly) | No packets received from that node | □ |
| 2 | Wait 120+ seconds (stale threshold) | Node marked LOST in /convoy.geojson | □ |
| 3 | Fetch /status — verify node still present with status: LOST (not removed) | Node in nodes array, status=LOST | □ |
| 4 | Bring node back into range (or power on) | New position packet received | □ |
| 5 | Fetch /convoy.geojson — verify status reverts to ACTIVE | status: "ACTIVE" | □ |
| 6 | Verify last_seen_ago shows fresh value (under 30s) | Age reset to near zero | □ |

| PASS if: Node transitions ACTIVE→LOST→ACTIVE correctly. Retained during LOST state. Age resets on recovery. | On FAIL: Check is_stale() threshold and node prune logic in _update_loop(). | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-06 Multi-Device Convoy State Suite F — Meshtastic ¦ CRITICAL

| Prerequisites | TC-F-02 passed. Minimum 3 Meshtastic devices active with GPS. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Bring all 3+ devices online with GPS fix | All nodes appear in /convoy.geojson | □ |
| 2 | Verify each device has distinct callsign (from Meshtastic long_name) | No duplicate callsigns | □ |
| 3 | Verify lead/tail assignment correct: lead = furthest along route direction | Lead and tail identified correctly | □ |
| 4 | Move lead vehicle forward — verify lead assignment follows | Lead role updates to correct vehicle | □ |
| 5 | Verify my_cart (configured via --my-node flag) marked is_my_cart=true in GeoJSON | My cart correctly identified | □ |

| PASS if: All devices tracked. Callsigns distinct. Lead/tail dynamically correct. My cart identified. | On FAIL: Verify --my-node argument passed at startup. Check _update_lead_tail() with 3+ nodes. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-F-07 Reconnect After USB Disconnect Suite F — Meshtastic ¦ HIGH

| Prerequisites | TC-F-01 passed. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | With backend running, physically unplug Meshtastic USB cable | Backend logs 'connection lost' | □ |
| 2 | Verify pubsub unsubscribe logged (no duplicate subscription warning) | Clean disconnect logged | □ |
| 3 | Verify backend does NOT crash — continues serving HTTP | Fetch /status still returns 200 | □ |
| 4 | Verify first reconnect attempt at ~1 second | Reconnect attempt logged at 1s | □ |
| 5 | Reconnect USB cable. Verify backend detects and reconnects. | 'reconnected successfully' in logs | □ |
| 6 | Fetch /convoy.geojson — verify positions resume updating after reconnect | Positions updating again | □ |

| PASS if: Backend survives disconnect. Reconnects automatically. HTTP serving uninterrupted. | On FAIL: Check _on_connection_lost() pubsub cleanup and watchdog backoff sequence. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Suite G — Full Field Operation

ENTRY CRITERIA: TC-E and TC-F suites complete. Minimum 3 vehicles. New Harmony UT staging area.

EQUIPMENT: All vehicles with Meshtastic nodes installed. Laptop with backend. All phones with Gaia and overlay.

TC-G-01 Pre-Departure System Check Suite G — Field ¦ CRITICAL

| Prerequisites | All vehicles at staging area. All Meshtastic nodes powered. Backend running. All phones on hotspot. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Start meshtastic_backend.py on laptop. Confirm all vehicles appear in /status | Unit count matches vehicle count | □ |
| 2 | Verify all vehicles have GPS fix (speed=0, valid lat/lon in /status) | All nodes show valid coordinates | □ |
| 3 | Verify all phones can reach /convoy.geojson and /status via hotspot | All devices confirm access | □ |
| 4 | Open Gaia on all phones — confirm convoy markers visible at staging area coordinates | Markers at correct staging location | □ |
| 5 | Open overlay on all phones — confirm HUD shows all units, server live badge | Overlays functional on all devices | □ |
| 6 | Confirm lead vehicle and tail vehicle identified correctly | Lead/tail assignment correct at staging | □ |
| 7 | Record baseline: unit count, GPS accuracy, battery levels, SNR values | Baseline recorded in test log | □ |

| PASS if: All vehicles tracked. All devices connected. Lead/tail assigned. Baseline recorded. | On FAIL: Resolve any connection issues before departure. Do not depart with less than 2/3 vehicles tracked. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-02 Convoy Movement — Position Updates Suite G — Field ¦ CRITICAL

| Prerequisites | TC-G-01 passed. Convoy in motion on New Harmony Rd. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Begin convoy movement NW from staging area | Convoy moving | □ |
| 2 | Monitor Gaia on lead vehicle — confirm own marker moves with vehicle | Lead marker tracks vehicle position | □ |
| 3 | Monitor overlay grid — confirm speed values increase from 0 as convoy moves | Speed > 0 for all active nodes | □ |
| 4 | Monitor overlay grid age column — confirm values stay under 30s for active nodes | Ages not staling during normal operation | □ |
| 5 | Confirm lead/tail roles update dynamically as convoy stretches out | Lead assignment follows furthest NW vehicle | □ |
| 6 | Monitor for any nodes dropping off unexpectedly — note distance at dropout | Dropout distances recorded | □ |

| PASS if: All nodes tracked during movement. Positions updating. Speed values correct. Lead/tail dynamic. | On FAIL: Note which vehicles drop first — likely range/terrain issue. Consider relay node placement. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-03 LoRa Range Test — Maximum Effective Range Suite G — Field ¦ HIGH

| Prerequisites | TC-G-02 in progress or completed. Lead vehicle continuing NW. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Extend lead vehicle further NW until position stops updating | Lead goes stale | □ |
| 2 | Record GPS position of lead vehicle at last successful packet | Max range position recorded | □ |
| 3 | Record GPS position of nearest relay/tail vehicle | Relay position recorded | □ |
| 4 | Calculate straight-line distance between positions | Max range in feet/miles recorded | □ |
| 5 | Note terrain between positions (open, trees, ridgeline, etc.) | Terrain context recorded | □ |
| 6 | Have lead vehicle return toward convoy — note distance at which contact restores | Recovery range recorded (may differ from dropout range) | □ |
| 7 | Verify CONTACT LOST banner fires and clears correctly on all observer phones | Alert system validated in field | □ |

| PASS if: Effective LoRa range measured. Recovery range measured. Alert system validated. | On FAIL: If range is insufficient for 3-mile convoy: evaluate relay node placement at convoy midpoint. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-04 Overlay Usability While in Motion Suite G — Field ¦ HIGH

| Prerequisites | TC-G-02 in progress. Vehicles moving. Observer (non-driver) evaluating overlay. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Observer reads HOTEL-10 speed from overlay at 15 mph movement | Speed value legible from moving vehicle | □ |
| 2 | Observer switches from CONVOY to MY CART view while moving | Toggle responsive in vehicle | □ |
| 3 | Observer reads proximity card values (AHEAD/BEHIND distances) | Proximity legible at speed | □ |
| 4 | Observer locates a specific callsign in grid HUD while moving | Grid scannable in 5 seconds | □ |
| 5 | Observer taps a grid row to open detail panel while moving | Detail panel opens — touch target adequate | □ |
| 6 | Driver attempts to glance at overlay at a stop — assess dashboard mounting viability | Mounting position notes recorded | □ |

| PASS if: All critical information readable in motion by observer. Interaction targets adequate. | On FAIL: Increase font size or reduce information density for next iteration. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-05 Hotspot Reliability Over 2-Hour Operation Suite G — Field ¦ HIGH

| Prerequisites | TC-G-02 in progress. 2-hour endurance. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Record hotspot connected device count at T=0 | All phones confirmed on hotspot | □ |
| 2 | At T=30 min: verify all phones still connected and receiving updates | All devices active at 30 min | □ |
| 3 | At T=60 min: check server rev count — confirm it has been incrementing | Rev count increasing as expected | □ |
| 4 | At T=90 min: verify no devices have dropped off hotspot | All devices still connected | □ |
| 5 | At T=120 min: final check — all systems operational | No degradation after 2 hours | □ |
| 6 | Note any hotspot disconnects or reconnects in the log | Interruptions documented | □ |

| PASS if: All devices maintain hotspot connection for 2-hour operation. No service interruption. | On FAIL: If hotspot drops: check laptop power management settings. Disable sleep when hotspot active. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-06 Simulated Vehicle Breakdown — LOST Contact Scenario Suite G — Field ¦ HIGH

| Prerequisites | TC-G-02 complete. All vehicles at staging or known positions. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Designate a test vehicle to simulate breakdown — have it power off Meshtastic node | Node goes silent | □ |
| 2 | All observer phones: verify CONTACT LOST banner appears within 2.5 minutes (120s + refresh cycle) | Banner fires on all observer devices | □ |
| 3 | Verify lost vehicle callsign appears in banner list | Correct callsign identified | □ |
| 4 | Verify lost vehicle marker turns red in Gaia on all phones | Red marker on all Gaia instances | □ |
| 5 | Test vehicle powers on Meshtastic node. Send position. | Node recovers | □ |
| 6 | Verify CONTACT LOST banner clears within 30 seconds of recovery packet | Alert clears on all devices | □ |
| 7 | Verify marker returns to original color in Gaia | Color restored | □ |

| PASS if: Lost contact detected within 2.5 min. All devices alert. Recovery clears alert. Consistent across all phones. | On FAIL: If some phones alert and others don't: check Gaia refresh intervals on individual devices. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-07 Backend Restart Recovery in Field Suite G — Field ¦ MEDIUM

| Prerequisites | TC-G-02 in progress. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Stop and restart meshtastic_backend.py while convoy is moving | Restart takes under 15 seconds | □ |
| 2 | Verify all phones show SRV DOWN badge during restart | Down state on all devices | □ |
| 3 | Verify all phones automatically recover to SRV LIVE after restart completes | Live state restored | □ |
| 4 | Verify GeoJSON resumes updating with correct node positions after restart | No stale or incorrect positions after restart | □ |

| PASS if: Backend restart takes under 15s. All clients recover automatically. Data integrity maintained. | On FAIL: Check startup sequence and ensure Meshtastic reconnects quickly after backend restart. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

TC-G-08 End-of-Mission Data Review Suite G — Field ¦ LOW

| Prerequisites | TC-G mission complete. Convoy returned to staging. |
| --- | --- |

| # | Action | Expected Observation | ✓ |
| --- | --- | --- | --- |
| 1 | Fetch final /status from all devices — confirm unit count matches | Consistent unit count | □ |
| 2 | Review server logs for any errors, warnings, or unexpected packet types | Log review complete | □ |
| 3 | Document: effective LoRa range, worst-case update latency, longest dropout, any crashes | Field performance data recorded | □ |
| 4 | Team debrief: rate overall system readiness on 1-5 scale. Note top 3 issues. | Debrief notes recorded | □ |
| 5 | Determine if Gate 1 is confirmed for operational deployment or needs another iteration | GO / NO-GO decision recorded | □ |

| PASS if: Performance data captured. Debrief complete. GO/NO-GO decision made. | On FAIL: Review issues against gap analysis. Prioritize top 3 for next development sprint. | Result: □ PASS □ FAIL □ BLOCKED Tester: ___________________ Date/Time: ________________ Notes: ____________________ |
| --- | --- | --- |

# Test Results Summary

Complete this table after each test session.

| ID | Test Name | Priority | Result | Notes |
| --- | --- | --- | --- | --- |
| TC-A-01 | GeoJSON Server Starts | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-02 | GeoJSON Endpoint Valid | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-03 | Status Endpoint JSON | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-04 | Phone Reaches Server | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-05 | Gaia Import Succeeds | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-06 | Markers on Terrain | CRITICAL | □ PASS □ FAIL □ SKIP | ← GATE DECISION |
| TC-A-07 | Marker Popup Telemetry | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-08 | Markers Update | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-A-09 | LOST Node Turns Red | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-A-10 | onX Validation | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-B-01 | Gaia Export Package | HIGH | □ PASS □ FAIL □ SKIP | Only if TC-A fails |
| TC-B-02 | Tile File Valid | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-B-03 | Tile to App Sandbox | HIGH | □ PASS □ FAIL □ SKIP | ← B1 GATE DECISION |
| TC-B-04 | GeoJSON Over Tiles | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-C-01 | TacticalMap Renders | MEDIUM | □ PASS □ FAIL □ SKIP | Baseline always |
| TC-C-02 | HUD Operational | MEDIUM | □ PASS □ FAIL □ SKIP |  |
| TC-D-01 | Backend All Endpoints | CRITICAL | □ PASS □ FAIL □ SKIP | Run early |
| TC-D-02 | CORS All Responses | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-D-03 | GeoJSON Schema | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-D-04 | Stale Node Retained | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-D-05 | Lead/Tail Sort | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-D-06 | Backoff Reconnect | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-D-07 | COT Type Stored | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-D-08 | Battery via TELEMETRY | MEDIUM | □ PASS □ FAIL □ SKIP |  |
| TC-E-01 | Overlay Transparent | CRITICAL | □ PASS □ FAIL □ SKIP | After TC-A passes |
| TC-E-02 | Grid HUD 20 Nodes | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-E-03 | View Toggle | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-E-04 | LOST Banner | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-E-05 | Server Badge States | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-E-07 | Live /status Polling | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-F-01 | Meshtastic Connect | CRITICAL | □ PASS □ FAIL □ SKIP | Requires hardware |
| TC-F-02 | POSITION_APP Updates | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-F-05 | Node Recovery | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-F-06 | Multi-Device Convoy | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-F-07 | USB Reconnect | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-G-01 | Pre-Departure Check | CRITICAL | □ PASS □ FAIL □ SKIP | Field day |
| TC-G-02 | Convoy Movement | CRITICAL | □ PASS □ FAIL □ SKIP |  |
| TC-G-03 | LoRa Range Test | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-G-06 | Lost Contact Scenario | HIGH | □ PASS □ FAIL □ SKIP |  |
| TC-G-08 | End-of-Mission Review | LOW | □ PASS □ FAIL □ SKIP |  |

| Overall Gate 1 Decision | □ PATH A CONFIRMED — proceed to live integration |  |
| --- | --- | --- |
|  | □ GATE 1 FAILED — proceed to Gate 2 (Suite B) |  |
| Overall Gate 2 Decision | □ PATH B1 VIABLE — begin tile-backed app development |  |
|  | □ GATE 2 FAILED — deploy Path B2 (Tracks Only) |  |
| Field GO/NO-GO | □ GO — deploy for operational use |  |
|  | □ NO-GO — issues to resolve: _______________ |  |
| Tester |  |  |
| Date |  |  |
| Firmware Version |  |  |
| Hardware |  |  |