DAY 2 — STEP-BY-STEP INSTRUCTIONS

Python-to-Mesh Connection & GPS Verification

Goal: Establish reliable Python BLE connection to T1000-E and verify GPS data flows into the Python tracker. ¦ Tasks: 2.1 – 2.6 ¦ Total Est. Time: ~100 minutes ¦ Deliverables: 3 verified outputs + GitHub commit

## Day 1 Status — What Was Already Completed

The following was completed on Day 1 (March 2, 2026) and is the baseline for Day 2.

| # | Task | Status |
| --- | --- | --- |
| 1 | GitHub account, repo, Git, SSH key | ☑ Done |
| 2 | Python 3.14, venv, dependencies installed | ☑ Done |
| 3 | Project scaffolding — all source files generated | ☑ Done |
| 4 | Meshtastic radio configuration (T1000-E × 2) | ☑ Done |
| 5 | Initial commit pushed to GitHub | ☑ Done |
| 6 | Mesh communication verified — text messaging works | ☑ Done |
| 7 | GPS position broadcasting verified via ATAK | ☑ Done |
| 8 | BLE connection from PC confirmed (meshtastic --ble-scan) | ☑ Done |

## Known Issues (Documented on Day 1)

These issues are already documented. Be aware of them as you proceed.

| Issue | Impact | Workaround |
| --- | --- | --- |
| Meshtastic phone apps (iOS & Android) don't render remote node positions on mesh map | Does not affect Python tracker — app bug only | Use ATAK for visual map; Python tracker reads positions directly via BLE |
| T1000-E BLE disconnect/reconnect loop on some Android devices | Secondary Android only; primary devices stable | Disable battery optimization; manually disconnect before leaving BLE range |
| Windows BLE + Python CLI sometimes fails to connect | Intermittent on Windows | Retry connection; close Web Client tabs first; ensure only one BLE client at a time |

# Prerequisites — Verify Before Starting

| ⚠ IMPORTANT | Complete ALL THREE before opening Git Bash. These are taken directly from the Implementation Schedule. Skipping any one is the most common cause of BLE connection failure. |
| --- | --- |

| # | Prerequisite (from Schedule) |
| --- | --- |
| 1 | Both T1000-E devices powered on and outdoors (GPS lock needed before any packets will appear) |
| 2 | iPhone disconnected from T1000-E before Python connects — only one BLE client at a time is allowed |
| 3 | Git Bash open with venv activated in project directory (you should see (venv) at the start of the prompt) |

Activate the Virtual Environment (if not already done)

In Git Bash, run:

cd ~/convoyprojectv4

source venv/Scripts/activate

You should see (venv) prepended to your prompt before continuing.

TASK 2.1 — Verify Python BLE Connection to T1000-E Owner: FRED ¦ Est. Time: 15 minutes ¦ Source: Schedule §Day 2, Task 2.1

What You Are Doing

Confirm the Python Meshtastic library can discover and connect to your T1000-E radio over BLE. This is the foundation for every other task today — nothing works until BLE connects successfully from Python.

Step-by-Step

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Confirm both T1000-E devices are powered on and outdoors. Check that the LED shows GPS lock (solid, not blinking). Allow 2–5 minutes warmup if just powered on. | FRED | 2–5 min |
| 2 | Confirm iPhone / any other BLE client is disconnected from the T1000-E. Only one client can hold a BLE connection at a time. | FRED | 1 min |
| 3 | In Git Bash (venv active), scan for BLE devices to find the T1000-E device name and MAC address. | FRED | 2 min |
| 4 | Note the device name (e.g. Lead_8e42) and the MAC address from the scan output — you will need both. | FRED | 1 min |
| 5 | Run the quick Python BLE connection test using the device name. Watch for node info to print in the console. | FRED | 3 min |
| 6 | If connection fails with device name, retry using the MAC address from step 4 (see fallback command below). | FRED | 3 min |
| 7 | Confirm node info and node list print successfully — BLE link is live. Take a screenshot. | FRED | 2 min |

Commands

BLE scan:

meshtastic --ble-scan

Quick BLE connection test (replace Lead_8e42 with your device name from scan):

python -c "from src.mesh_interface import MeshInterface; m = MeshInterface({'connection_type': 'ble', 'ble_device_name': 'Lead_8e42'}); m.connect(); print(m.get_node_list()); m.disconnect()"

Fallback — connect by MAC address if name fails:

python -c "from src.mesh_interface import MeshInterface; m = MeshInterface({'connection_type': 'ble', 'ble_address': 'XX:XX:XX:XX:XX:XX'}); m.connect(); print(m.get_node_list()); m.disconnect()"

| NOTE | Windows BLE + Python CLI sometimes fails to connect is a documented Known Issue. Fix: Retry the connection. Close any Meshtastic Web Client browser tabs first. Ensure only one BLE client is connected at a time. |
| --- | --- |

✓ PASS CRITERIA

☐ Node info and node list print to the console without errors

☐ At least one T1000-E node entry is visible in the list

TASK 2.2 — Run test_connection.py — Confirm Node Info and Node List Owner: BOTH (FRED runs, CLAUDE assists) ¦ Est. Time: 15 minutes

What You Are Doing

Run the project's dedicated connection test script. This performs a full BLE connect, retrieves the node info for the connected T1000-E, prints the complete list of all nodes the radio knows about on the mesh, then disconnects cleanly. It validates that mesh_interface.py works end-to-end.

Step-by-Step

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Confirm venv is still active — (venv) is visible at the start of the Git Bash prompt. | FRED | < 1 min |
| 2 | Run the connection test script. | BOTH | 1 min |
| 3 | Wait for the script to complete. It connects, queries node info and node list, then disconnects. | BOTH | 2 min |
| 4 | Verify the T1000-E you are connected to appears with its correct longName and node ID. | BOTH | 1 min |
| 5 | Verify the second T1000-E also appears in the node list (if it is in range and has synced). | BOTH | 1 min |
| 6 | Screenshot the full console output and save it to the docs/ folder. | FRED | 2 min |
| 7 | If the script errors, note the exact error message and proceed to Task 2.5 (debug). Otherwise continue to 2.3. | BOTH | — |

Command

python tests/test_connection.py

Expected Console Output

Connecting to T1000-E via BLE...

Connected!

My node: {'id': '!abc12345', 'longName': 'Lead', 'shortName': 'L1', ...}

Nodes on mesh: [{'id': '!abc12345', ...}, {'id': '!def67890', ...}]

Connection test PASSED

| TIP | If only one node appears in the node list, the second T1000-E may not have synced yet. Meshtastic nodes exchange node lists periodically, not instantly. Wait 60 seconds and rerun. |
| --- | --- |

✓ PASS CRITERIA

☐ Script exits cleanly

☐ Console shows 'Connection test PASSED'

☐ Both T1000-E nodes visible in the node list

TASK 2.3 — Run test_gps.py — Listen for POSITION_APP Packets (2 Minutes) Owner: BOTH (FRED runs, CLAUDE assists) ¦ Est. Time: 15 minutes

What You Are Doing

Listen on the BLE connection for incoming POSITION_APP packets — these are the raw GPS payloads broadcast by each T1000-E radio at a configurable interval (typically every 30–60 seconds). Confirming that Python receives these packets is the critical proof that GPS data flows into the tracker.

| ★ CRITICAL | This is the most important verification in Day 2. Do not skip or abbreviate it. |
| --- | --- |

Step-by-Step

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Confirm both T1000-E devices are still outdoors with GPS lock. GPS packets will NOT arrive from an indoor device. | FRED | 1 min |
| 2 | Run the GPS listener test script. It will listen for POSITION_APP packets for 2 minutes (120 seconds). | BOTH | 1 min |
| 3 | Wait the full 2 minutes. GPS packets broadcast every 30–60 seconds — do not stop the script early. | BOTH | 120 sec |
| 4 | Watch the console for incoming packets. Each line printed = one GPS packet received from one node. | BOTH | During run |
| 5 | Verify at least ONE packet arrives from EACH T1000-E device during the 2-minute window. | BOTH | 1 min |
| 6 | Verify the lat/lon values are geographically plausible for your location (New Harmony, UT area). | BOTH | 1 min |
| 7 | Screenshot the console output showing packets received and save to docs/. | FRED | 2 min |

Command

python tests/test_gps.py

Expected Console Output

Listening for POSITION_APP packets for 120 seconds...

[00:00:34] Node !abc12345 (Lead) -> lat: 37.4521, lon: -113.3498, alt: 915m, speed: 0 mph

[00:01:07] Node !def67890 (Tail) -> lat: 37.4478, lon: -113.3521, alt: 910m, speed: 0 mph

[00:01:41] Node !abc12345 (Lead) -> lat: 37.4521, lon: -113.3498, alt: 915m, speed: 0 mph

Test complete. Received 3 packets from 2 nodes. GPS VERIFIED.

| NO PACKETS? | GPS packets only transmit every 30–60 seconds — you must wait the full 2 minutes. If nothing arrives: (1) Confirm devices are OUTDOORS. GPS does not work indoors. (2) Confirm only one BLE client is connected. (3) If still nothing, power-cycle the radio and retry. If packets arrive from only one node, the second may need longer to sync — retry after 2 more minutes. |
| --- | --- |

✓ PASS CRITERIA

☐ At least 1 POSITION_APP packet received from each T1000-E within the 2-minute window

☐ Coordinates are geographically correct (New Harmony / St. George, UT area)

TASK 2.4 — Run test_kml.py — Generate Test KML and Open in Google Earth Owner: BOTH (FRED runs, CLAUDE assists) ¦ Est. Time: 15 minutes

What You Are Doing

Run the KML generation test. The script reads the latest GPS positions received from the T1000-E radios, generates a test_convoy.kml file, and saves it to the output/ directory. You then open the KML in Google Earth to visually confirm that the node positions are plotted correctly on the map — this validates the full data-to-display pipeline.

Step-by-Step

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Run the KML generation test script. | BOTH | 1 min |
| 2 | Confirm the output file was created at output/test_convoy.kml. | BOTH | 1 min |
| 3 | Open Google Earth Pro on the laptop. | FRED | 1 min |
| 4 | In Google Earth: File > Open > navigate to and select output/test_convoy.kml. | FRED | 1 min |
| 5 | Verify map pins/icons appear at the correct GPS coordinates for each node. | BOTH | 2 min |
| 6 | Verify node labels match the T1000-E callsigns (e.g. Lead, Tail). | BOTH | 2 min |
| 7 | Screenshot the Google Earth view showing both pins and save to docs/. | FRED | 2 min |

Commands

Run test:

python tests/test_kml.py

Verify file exists:

ls output/

# Expected: test_convoy.kml

| TIP | If Google Earth Pro is not installed, it is available free from google.com/earth/about/versions. As an alternative, you can drag the .kml file directly into a Google Maps browser window to view it. |
| --- | --- |

✓ PASS CRITERIA

☐ File output/test_convoy.kml exists on disk

☐ Google Earth displays both node pins at correct real-world positions

☐ Callsign labels are correct (Lead, Tail)

TASK 2.5 — Debug Any BLE Connection Issues on Windows Owner: BOTH (FRED runs, CLAUDE diagnoses and fixes) ¦ Est. Time: 30 minutes (if needed)

| SKIP IF PASSING | This task only applies if issues were observed during Tasks 2.1–2.4. If all prior tasks passed, skip directly to Task 2.6. |
| --- | --- |

What You Are Doing

Systematically diagnose and resolve any BLE connection problems encountered during Tasks 2.1–2.4.

Troubleshooting Reference

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| --ble-scan returns no devices | Bluetooth off, or another client connected | Toggle BT off/on in Windows Settings. Disconnect phone app. |
| Connection timeout after ~10s | Device out of range or busy | Move laptop closer (< 3m to radio). Power-cycle radio. |
| 'Device not found' by name | Name not advertising or changed | Use MAC address from --ble-scan instead of name. |
| Connects then immediately drops | Meshtastic Web Client tab still open | Close ALL browser tabs. Retry. |
| OSError or WinError on connect | Windows BLE stack stale state | Restart Bluetooth in Device Manager (disable then enable). |

Step-by-Step Debug Procedure

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Close all Meshtastic Web Client browser tabs and any other BLE applications. | FRED | 1 min |
| 2 | In Windows Settings > Bluetooth & devices, toggle Bluetooth OFF, wait 5 seconds, toggle ON. | FRED | 1 min |
| 3 | Retry the BLE scan: meshtastic --ble-scan | FRED | 1 min |
| 4 | If scan finds device but connection still fails: open Device Manager, expand Bluetooth, right-click BT adapter, Disable > Enable. | FRED | 2 min |
| 5 | Retry the connection using MAC address (not name) from the scan output. | BOTH | 2 min |
| 6 | If error occurs in Python: copy the exact error text and share with Claude for diagnosis and code fix. | BOTH | variable |
| 7 | Claude applies any necessary code fixes to mesh_interface.py or related files. | CLAUDE | variable |
| 8 | Retest Task 2.1 after any fix to confirm BLE connection is stable. | BOTH | 5 min |

✓ PASS CRITERIA

☐ BLE connection completes without error

☐ Tasks 2.1–2.4 pass after any fixes are applied

☐ All issues and their fixes documented in docs/

TASK 2.6 — Commit Test Results and Any Code Fixes to GitHub Owner: FRED ¦ Est. Time: 10 minutes ¦ Source: Schedule §Day 2, Task 2.6

What You Are Doing

Commit all Day 2 test results, screenshots saved to docs/, and any code fixes made during Task 2.5 to the GitHub repository. This records the verified Day 2 baseline and enables clean rollback if future changes break the BLE connection path.

Step-by-Step

| # | Action | Owner | Time |
| --- | --- | --- | --- |
| 1 | Confirm you are in the project root directory. | FRED | < 1 min |
| 2 | Check git status to see which files are new or changed. | FRED | 1 min |
| 3 | Stage all changes — test results, screenshots, any code fixes. | FRED | 1 min |
| 4 | Commit with a descriptive Day 2 message. | FRED | 1 min |
| 5 | Push to GitHub. | FRED | 1 min |
| 6 | Open GitHub in browser and verify the commit appears on the repo main page with the correct message. | FRED | 2 min |

Commands

cd ~/convoyprojectv4

git status

git add .

git commit -m "Day 2: BLE connection verified, GPS packets confirmed, KML test passed"

git push origin main

| TIP | If git push is rejected with 'non-fast-forward' error, run: git pull origin main --rebase then retry the push. This happens when a file was edited directly on GitHub. |
| --- | --- |

✓ PASS CRITERIA

☐ git push completes without error

☐ Commit is visible on the GitHub repo page with the Day 2 message

# Day 2 Deliverables Summary

These are the three deliverables specified in the Implementation Schedule for Day 2.

| Deliverable | What It Proves | Saved To |
| --- | --- | --- |
| Confirmed POSITION_APP packets received in Python | Full GPS data path works end-to-end: radio → BLE → Python | Screenshot in docs/ |
| test_convoy.kml viewable in Google Earth | KML export pipeline works — node positions correct on map | output/test_convoy.kml |
| Console output screenshots saved to docs/ | Permanent record of Day 2 verification for each task | docs/ folder |

# End-of-Day 2 Checklist

| VERIFY ALL | Do not begin Day 3 (Core Module Integration Testing) until every item below is checked. |
| --- | --- |

| Task | Check |
| --- | --- |
| 2.1 | ☐ meshtastic --ble-scan finds the T1000-E device |
| 2.1 | ☐ Python BLE connection test prints node info and node list without errors |
| 2.2 | ☐ test_connection.py runs and prints 'Connection test PASSED' |
| 2.2 | ☐ Both T1000-E nodes visible in node list from test_connection.py |
| 2.3 | ☐ test_gps.py receives at least 1 POSITION_APP packet from each T1000-E within 2 minutes |
| 2.3 | ☐ GPS coordinates from test_gps.py are geographically correct |
| 2.4 | ☐ output/test_convoy.kml file exists on disk |
| 2.4 | ☐ KML opens in Google Earth with both node pins at correct locations and correct callsign labels |
| 2.5 | ☐ Any BLE issues encountered are documented in docs/ with symptoms and fixes applied |
| 2.6 | ☐ All Day 2 results, screenshots, and code fixes committed to GitHub |
| 2.6 | ☐ Commit visible on GitHub repo main page with Day 2 message |

## What Comes Next — Day 3: Core Module Integration Testing

Day 3 tests each Python module individually, then verifies they work together. Preview of Day 3 tasks:

| # | Task | Owner | Est. Time |
| --- | --- | --- | --- |
| 3.1 | Test mesh_interface.py — BLE connect, listen, disconnect cycle | BOTH | 30 min |
| 3.2 | Test gps_handler.py — feed live positions, verify haversine distance | BOTH | 30 min |
| 3.3 | Test convoy_manager.py — verify spread calculation with 2 nodes | BOTH | 30 min |
| 3.4 | Test kml_export.py — generate KML from live node positions | BOTH | 20 min |
| 3.5 | Run unit tests: python -m pytest tests/ | BOTH | 15 min |
| 3.6 | Fix any bugs found during testing | CLAUDE | 30 min |
| 3.7 | Commit fixes to GitHub | FRED | 10 min |