CONVOY TRACKER

User Manual — V2

March 20, 2026  |  Android  |  Off-Grid Convoy Coordination

# What is Convoy Tracker?

Convoy Tracker is an Android app for coordinating multi-vehicle group rides without cellular or internet connectivity. It uses Meshtastic mesh radio hardware to share GPS positions and status between all vehicles in a convoy. No phone signal required — everything works via LoRa radio.

Key benefit: Every rider's position is visible to every other rider in real time, even in remote areas with no cell service.

# Hardware Requirements

Each vehicle needs one Meshtastic radio device. Supported hardware:

- SenseCAP T1000-E — primary recommended device (compact, weatherproof)

- T-Beam v2 — alternative with display

- Any Meshtastic-compatible LoRa device running firmware 2.6+

The radio connects to your Android phone via Bluetooth. The Convoy app communicates with the radio through the Meshtastic protocol.

# Installation

## New Device Setup

- Enable Unknown Sources: Settings → Security → Install unknown apps → allow for your email/file manager app

- Open the Convoy APK attachment in your email

- Tap Install when prompted

- Open Convoy Tracker from your app drawer

## Radio Pairing

- Turn on your Meshtastic radio

- Open Convoy Tracker

- Tap the Meshtastic connections icon (top of screen)

- Select your radio from the Bluetooth scan list

- Wait for CONNECTED status

# User Roles

There are two user types: ORGANIZER (creates rides, configures radios) and RIDER (joins rides, has radio configured automatically).

## Organizer

The organizer creates the ride, generates the encrypted channel, configures their own radio, and distributes the ride config to riders via email. One organizer per ride.

## Rider

The rider receives the ride config email, imports it into Convoy Tracker, connects their radio, and applies the config. The radio is configured automatically — no manual settings required.

# Organizer: Creating a Ride

## Step 1 — Create the Ride

- Open Convoy Tracker

- Tap the CONVOY header to open the menu

- Tap EVENT / RIDE → Create a Ride

- Enter ride name, date, and description

- The app generates a unique encrypted channel name (e.g. CONVOY-HD26) and AES-256 encryption key

- Tap SAVE RIDE

## Step 2 — Configure Your Radio

- Connect your radio via Bluetooth

- Tap the CONVOY header → Apply Ride Info to Radio

- Select RIDE tab

- Select your newly created ride

- Review the REVIEW CHANGES screen — verify channel name is correct

- Tap PROCEED TO UPDATE

- Wait for Stage 1 (60 seconds — radio rebooting)

- When WRITE CHANNEL + PSK button turns blue, tap it

- Wait for Stage 2 (60 seconds — channel write reboot)

- Tap PROCEED TO VERIFY when green

- Verify screen shows all 40 fields ✓ — radio is configured

## Step 3 — Send Ride to Riders

- Tap CONVOY header → EVENT / RIDE → Transfer Event/Ride Details via Email

- Search for your ride by organizer name or scroll to find it

- Select your ride (checkmark appears)

- Tap SEND VIA EMAIL

- Select your email app from the share sheet

- Send to all riders

# Rider: Joining a Ride

## Step 1 — Import the Ride

- Open the ride email on your Android phone

- Tap the .convoy attachment

- Convoy Tracker opens automatically

- Next time you open the CONVOY menu, a splash screen confirms: NEW RIDE IMPORTED: <ride name>

- The ride now appears in Apply Radio → RIDE tab

## Step 2 — Configure Your Radio

- Connect your radio via Bluetooth

- Tap CONVOY header → Apply Ride Info to Radio

- Select RIDE tab

- Select the imported ride

- Tap PROCEED TO UPDATE

- Follow the same Stage 1 / Stage 2 wait process as the organizer

- Verify screen confirms all 40 fields ✓

Important: The radio is now configured with the ride's encrypted channel. You can now communicate with all other riders on the same ride.

# The Map Screen

The main screen shows a Leaflet map with all convoy nodes plotted. The transparent HUD overlay shows convoy status.

## Button Bar

| GROUP | Shows all carts — span, lead, tail, active/lost count |
| --- | --- |

| MY CART | Shows your cart details — heading, battery, altitude, speed |
| --- | --- |

| HIDE | Hides the HUD overlay — map only view |
| --- | --- |

| GEAR (⚙) | Opens map settings and layer controls |
| --- | --- |

## HUD Panels

GROUP panel shows: Ride title, cart count (active/lost), convoy span in miles, lead cart callsign, tail cart callsign.

MY CART panel shows: Your callsign, heading in degrees, battery percentage, altitude in feet, current speed, and distance traveled.

# Radio Config — Apply Radio

## MASTER mode

Applies the master radio config to any radio. Resets the radio to the standard convoy baseline: region US, LONG_FAST preset, 60s GPS broadcast, AES-256 Convoy channel. Use this to restore a radio after a ride or set up a new radio.

## RIDE mode

Applies a specific ride config. Uses the master baseline plus the ride's unique channel name and AES-256 encryption key. Only radios configured for the same ride can communicate with each other.

## The Two-Stage Process

Stage 1: Binary install (38 radio config fields, one reboot, 60 seconds) Stage 2: Channel write (channel name + PSK, one reboot, 60 seconds) Total time: approximately 2.5 minutes

When Stage 1 countdown completes and the radio reconnects, the WRITE CHANNEL + PSK button turns blue. Do not tap it until the radio has reconnected and the button is blue.

# Archive and Restore

Before every radio config apply, Convoy Tracker automatically saves a backup of the current radio state. To restore a previous config:

- Tap CONVOY header → Restore Prior Config Post Ride

- Archives are listed newest first for your connected radio

- Select the archive you want to restore

- Tap RESTORE THIS ARCHIVE

- Wait 60 seconds for the radio to reboot

- Radio is restored to the selected backup

# Bluetooth Reconnect

After a radio reboot, it may not reconnect automatically. If the WRITE CHANNEL or PROCEED TO VERIFY button does not turn blue after 60 seconds:

- Pull down the Android notification shade

- Tap Bluetooth OFF

- Wait 3 seconds

- Tap Bluetooth ON

- Wait for the radio to reconnect (green dot in Meshtastic)

- The button will enable once connected

# Troubleshooting

## Radio shows yellow lock icon (encryption not active)

This means the channel write did not complete correctly. Run Apply Radio again. Make sure to wait for the full 60-second Stage 2 countdown and tap PROCEED TO VERIFY only after the radio reconnects.

## Verify screen shows field failures

Some fields did not write correctly. Note which fields failed and run Apply Radio again. If the same fields fail repeatedly, the master config may need to be recaptured from a known-good radio.

## Ride not appearing in RIDE tab

The ride import may not have completed. Open the CONVOY menu — if a splash screen appears, the import is processing. If no splash, open Transfer Ride and resend the ride to yourself.

## Cannot press RECORD button

Cart disconnect messages may be overlaying the RECORD button. Dismiss any active status messages or rotate the screen to reposition the overlay.

# Radio Config Reference

Default master config values applied to all convoy radios:

| Region | US (915 MHz) |
| --- | --- |

| Modem Preset | LONG_FAST |
| --- | --- |

| Hop Limit | 3 |
| --- | --- |

| TX Power | 27 dBm |
| --- | --- |

| GPS Broadcast | 60 seconds |
| --- | --- |

| Smart Position | Enabled — 100m minimum distance |
| --- | --- |

| Display Units | Imperial (miles, feet, mph) |
| --- | --- |

| Channel Name | CONVOY (master) or CONVOY-XXXX (ride-specific) |
| --- | --- |

| Encryption | AES-256, unique key per ride |
| --- | --- |

| Position Precision | 32 (full precision — exact location shared) |
| --- | --- |

CONVOY TRACKER  |  User Manual V2  |  March 20, 2026