CONVOY TRACKER

Day 17–18 Close-Out  |  Day 19–20 Plan

March 19–20, 2026  |  Branch: feature/convoy-event-ride  |  kixazz25/Meshtastic-Android

MILESTONE ACHIEVED — March 19, 2026 Binary cfg engine proven end-to-end. Ride config applied to factory-reset radio. Channel + PSK written. 38 fields verified. Zero manual radio knowledge required.

# Day 17–18 Completed Items

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 1 | ConvoyProfileBuilder.kt | Binary .cfg built from WorkingConfig JSON. No radio required. All 4 config groups serialised to DeviceProfile protobuf. | DONE TODAY |
| 2 | Rules engine wiring | WorkingConfig flows from rules engine through builder to InstallProfileUseCase. Four sources merged correctly. | DONE TODAY |
| 3 | Binary install path | Replaced 4-piece write strategy (setConfig x3 + setChannels) with single InstallProfileUseCase call. One write, one reboot. | DONE TODAY |
| 4 | Channel write after install | setChannels() called in ReconnectWait Stage 1 PROCEED button after radio reconnects from binary install reboot. | DONE TODAY |
| 5 | ReconnectWait two-stage gated | Stage 1: 60s wait + WRITE CHANNEL button. Stage 2: PROCEED TO VERIFY button. Both enabled only when connected after gate. | DONE TODAY |
| 6 | ViewModel scoping fix | Shared ConvoyViewModel passed through nav graph to ReconnectWait. WorkingConfig no longer null on screen entry. | DONE TODAY |
| 7 | Verify screen — errors only | 38 fields compared. Only failures shown. Group summary always shown. Module fields marked informational. | DONE TODAY |
| 8 | master.cfg recapture | All three assets resynced from verified radio: master.cfg, master_config.json, convoy_apply_list.json. Committed. | DONE TODAY |
| 9 | RIDE mode end-to-end test | Ride config applied to factory-reset radio. Ride-specific channel name and AES256 PSK written and verified. PASS. | DONE TODAY |
| 10 | channel_url added to DeviceProfile | ChannelSet encoded as base64 channel_url in DeviceProfile binary. | DONE TODAY |

# Day 17–18 Items NOT Completed — Carry Forward

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 1 | Archive Restore Screen | ConvoyRestoreFromArchiveScreen.kt — list archives sorted by date, select, install, verify. | DEFERRED |
| 2 | GPS Recording Persistence | ConvoyGpsService.kt foreground service, KML recording, battery optimization exemption. | DEFERRED |
| 3 | Ride Config Transfer | TRANSFER-01/02/03 — auto-email, manual send, import from email attachment. | DEFERRED |
| 4 | Verify screen Module fields | 17 module fields marked informational. Need moduleConfigFlow wired to Verify for real comparison. | OPEN |
| 5 | UI-CLEAN-01 through 04 | Remove post-capture nav, *.bak gitignore, Log.e debug statements, smart position slider. | DEFERRED |

# Day 19 — Open Issues from Day 18 Testing

These five issues were identified during end-to-end testing on March 19. Address in order before moving to new features.

## Issue 1 — Location and Position Precision fields not in JSON or Verify

Channel module_settings (position_precision and is_muted) are not captured in master_config.json and not written by the channel write. These appear in Meshtastic settings as Location toggle and Precise Location toggle. They live inside ChannelSettings.module_settings, a separate protobuf not currently read or written by the convoy flow.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 19-1 | Research module_settings protobuf | Find position_precision and is_muted field names in channel.proto. Confirm they are in ChannelSettings.module_settings. | TOMORROW |
| 19-2 | Add to ConvoyMasterConfig.kt | Add positionPrecision: Int and isMuted: Boolean fields to data class and JSON serialisation. | TOMORROW |
| 19-3 | Add to master_config.json capture | ConvoyMasterCaptureScreen reads and saves these fields. Recapture master_config.json. | TOMORROW |
| 19-4 | Add to WorkingConfig | Add fields to WorkingConfig data class in ConvoyRadioWriter.kt. | TOMORROW |
| 19-5 | Add to channel write | Include ModuleSettings(position_precision, is_muted) in ChannelSettings when writeChannel() fires. | TOMORROW |
| 19-6 | Add to Verify screen | Compare channel module_settings fields against WorkingConfig values. | TOMORROW |

## Issue 2 — Apply Radio Screen too busy — accordion collapse required

The PROCEED screen shows all field values expanded by default. Riders see too much information. Only the Long Name field should be visible. All config groups (LoRa, Channel, Device, Position, Display, Module) should be collapsed as accordion twisties — diagnostic tools, not primary UI.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 19-7 | Collapse all accordions by default | Set confirmLoraOpen, confirmChannelOpen, confirmDeviceOpen, confirmPositionOpen, confirmDisplayOpen, confirmModuleOpen all to false on init. | TOMORROW |
| 19-8 | Long Name field always visible | Keep Long Name text field above accordion section. This is the only field riders need to see. | TOMORROW |
| 19-9 | Show new channel name prominently | When RIDE mode: show new channel name and source clearly above accordions. Simple, not busy. | TOMORROW |

## Issue 3 — App does not persist if user navigates to Meshtastic connections screen

Navigating to the Meshtastic connections screen during the write flow destroys the ReconnectWait screen and kills the job. WorkingConfig is cleared. User must restart the process from the beginning.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 19-10 | Research SavedStateHandle for WorkingConfig | Determine if WorkingConfig can survive navigation events via SavedStateHandle or persistent storage. | TOMORROW |
| 19-11 | Store WorkingConfig to file before write | Write WorkingConfig JSON to a temp file before binary install. Restore if screen is recreated. | TOMORROW |
| 19-12 | Add resume detection to ReconnectWait | If WorkingConfig is null on entry, attempt to restore from temp file. | TOMORROW |

## Issue 4 — Manual BT toggle instruction needed

The automated BT toggle (adapter.disable/enable) silently fails on Android 13+. The ReconnectWait screen shows a BT toggle message but nothing happens. User must toggle BT manually. Screen must display clear instructions.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 19-13 | Replace BT toggle with manual instruction | After 60s wait, if not connected: show 'Please toggle Bluetooth OFF then ON in Android settings. PROCEED will enable when connected.' | TOMORROW |
| 19-14 | Remove reconnectDevice() call | Remove the broken adapter.disable/enable calls from UIViewModel.reconnectDevice() or replace with instruction-only flow. | TOMORROW |

## Issue 5 — Remove 60 second pause after channel write

Channel write (setChannels) does NOT trigger a radio reboot. The current Stage 2 60-second wait is unnecessary and wastes 60 seconds every time. Replace with a 3-second delay then proceed directly to Verify.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 19-15 | Replace Stage 2 60s wait with 3s delay | After channel write: delay(3000) then navigate directly to Verify. No countdown, no BT toggle, no gate. | TOMORROW |
| 19-16 | Simplify ReconnectWait to one stage | Stage 1 only: 60s wait for binary install reboot + BT manual instruction + WRITE CHANNEL button. After write: 3s delay + straight to Verify. | TOMORROW |

# Day 19 — March 20, 2026

Focus: Close all 5 open issues from Day 18 testing. Each fix is surgical and independent. Complete in order — Verify first so it can confirm each fix.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 1 | Issue 5 — Remove 60s Stage 2 wait | ReconnectWait: replace Stage 2 with 3s delay, straight to Verify. Quick win, test immediately. | TOMORROW |
| 2 | Issue 2 — Collapse accordions | All config groups collapsed by default. Long Name visible. Channel name prominent in RIDE mode. | TOMORROW |
| 3 | Issue 4 — Manual BT toggle instruction | Replace broken BT toggle with clear manual instruction text after 60s wait. | TOMORROW |
| 4 | Issue 1 — position_precision research | Read channel.proto. Find module_settings fields. Document before writing any code. | TOMORROW |
| 5 | Issue 1 — Add to data classes | Add positionPrecision + isMuted to ConvoyMasterConfig, WorkingConfig, channel write, Verify. | TOMORROW |
| 6 | Issue 1 — Recapture master assets | After data class changes: recapture master_config.json from radio. Commit all three assets. | TOMORROW |
| 7 | Issue 3 — WorkingConfig persistence | Research and implement SavedStateHandle or file-based persistence for WorkingConfig. | TOMORROW |
| 8 | Full end-to-end test | Factory reset radio. Apply MASTER. Apply RIDE. Verify passes all fields including position_precision. | TOMORROW |
| 9 | Commit and push | Clean commit with all Day 19 fixes. Tag as v2-beta-candidate if all tests pass. | TOMORROW |

# Day 20 — March 21, 2026

Focus: UI polish pass + GPS recording foundation + Archive Restore screen. V2 feature complete.

| # | TASK | DETAIL / NOTES | STATUS |
| --- | --- | --- | --- |
| 1 | UI-CLEAN-01 | Remove post-capture navigation to apply list in ConvoyMasterSuccess. | TOMORROW |
| 2 | UI-CLEAN-02 | Add *.bak to .gitignore. | TOMORROW |
| 3 | UI-CLEAN-03 | Remove Log.e debug statements from ConvoyNavigation. | TOMORROW |
| 4 | UI-CLEAN-04 | Smart position broadcast interval slider in settings panel. | TOMORROW |
| 5 | GPS-01 — ConvoyGpsService scaffold | Android foreground service skeleton. START/STOP controls in button bar. Verify survives backgrounding. | TOMORROW |
| 6 | GPS-02 — File name dialog | File name dialog moved to END RECORD. KML file named when recording stops. | TOMORROW |
| 7 | GPS-03 — Battery optimization | Request Tier 3 battery optimization exemption. Service not killed by Doze. | TOMORROW |
| 8 | Archive Restore Screen | ConvoyRestoreFromArchiveScreen.kt. List archives sorted by date. Select, install, verify. Wire as Process 3 in Apply Radio. | TOMORROW |
| 9 | Ride Config Transfer stub | TRANSFER-01: auto-email ride JSON on creation. TRANSFER-03: register as .json file handler, parse, add to local ride DB. | TOMORROW |
| 10 | V2 feature complete commit | Tag v2-feature-complete. All screens, all flows, all tests passing. | TOMORROW |

# Environment Reference

| Branch | feature/convoy-event-ride |
| --- | --- |
| Last Commit | MILESTONE — binary cfg engine + ride test pass |
| Build | ./gradlew assembleGoogleDebug |
| Install | adb -s 8624SBCEDF00001789 install -r app/build/outputs/apk/google/debug/app-google-debug.apk |
| Device | 8624SBCEDF00001789 ¦ Node: !1d5bdc79 ¦ tracker-t1000-e fw 2.6.11.60ec05e |
| Assets | app/src/main/assets/ — master.cfg, master_config.json, convoy_apply_list.json |
| Archives | files/convoy_backups/!1d5bdc79/ |

CONVOY TRACKER  |  Day 17–18 Close-Out / Day 19–20 Plan  |  March 19, 2026