CONVOY TRACKER

Radio Write Process — Design Document

Branch: feature/convoy-event-ride  |  Author: Fred Kix

Three processes — one write engine — five screens

# OVERVIEW

The radio write process applies a complete convoy configuration to a Meshtastic radio via Bluetooth. Three distinct processes use the same screen-by-screen write engine. Each screen handles one config group, executes the write, and monitors for reboot before advancing to the next screen. A final verify screen reads back all values and confirms the write was successful.

# THREE WRITE PROCESSES

## Process 1 — Apply Master Config

Applies the master_config.json to the radio. Used to standardize a radio to the base convoy configuration.

- Config source: master_config.json

- Channel name + PSK: from master_config.json

- Checklist rules applied: fields in apply list are written, others kept from radio

- Long name: from master config, editable before write

## Process 2 — Apply Ride Config

Applies a ride-specific configuration. Channel name and PSK come from the selected ride file.

- Config source: master_config.json + ride file override

- Channel name + PSK: from selected ride file

- Checklist rules applied: same as Process 1

- Long name: from working config, editable before write

## Process 3 — Restore from Archive

Restores a radio to a previously archived configuration. No rules engine — exact restore.

- Config source: selected archive file from convoy_backups/

- All values restored exactly — no overrides

- No channel/PSK override — original radio state completely restored

- Used for: post-ride restore, error recovery

# WRITE PROCESS FLOW — All Three Processes

## Phase 0 — Pre-Write (ConvoyApplyRadioScreen)

🔒 GATE: Radio must be connected via Bluetooth

🔒 GATE: For Process 2: ride must be selected from list

Actions performed:

- User selects operation: MASTER, RIDE, or RESTORE

- For RIDE: select ride from list sorted by date, today as default

- Confirmation accordion shown: Current / New Value / Rule for each field

- PROCEED TO UPDATE tapped:

- → Archive current radio state — saved to convoy_backups/<nodeId>/pre_write_<timestamp>.json

- → Build WorkingConfig from selected source

- → Store WorkingConfig in ConvoyViewModel

- → Navigate to Screen 1

## Screen 1 — Device Config (ConvoyDeviceConfigScreen)

🔒 GATE: PROCEED button: red + disabled when BT disconnected, green + enabled when connected

Fields displayed (read-only, pre-populated from working config):

- Long Name — from working config

- Node Role — CLIENT

- Managed Mode — false

- Serial Enabled — false

On PROCEED:

- Calls channelViewModel.setConfig(Config(device = ...))

- No reboot triggered

✓ Navigate immediately to Screen 2

## Screen 2 — LoRa Config (ConvoyLoRaConfigScreen)

🔒 GATE: PROCEED button: red + disabled when BT disconnected, green + enabled when connected

Fields displayed (read-only):

- Region — from working config (e.g. US)

- Modem Preset — from working config (e.g. LONG_FAST)

- Hop Limit — from working config

- TX Enabled — from working config

- TX Power — from working config (dBm)

On PROCEED:

- Calls channelViewModel.setConfig(Config(lora = ...))

⚠  REBOOT TRIGGERED — radio disconnects after LoRa write

- Button changes to: WAITING FOR RADIO…

- App monitors connectionState flow for reconnect

- CONTINUE button enables when BT reconnects

✓ User taps CONTINUE — navigate to Screen 3

## Screen 3 — Position Config (ConvoyPositionConfigScreen)

🔒 GATE: PROCEED button: red + disabled when BT disconnected, green + enabled when connected

Fields displayed (read-only — convoy values override Android defaults):

| Field | Convoy Value | Android Default | Source |
| --- | --- | --- | --- |
| Broadcast Interval | 5 seconds | 1 minute minimum | Convoy hardcoded |
| GPS Update Interval | 1 second | 8 seconds minimum | Convoy hardcoded |
| Smart Position | Enabled | Varies | Convoy hardcoded |
| Smart Min Interval | 3 seconds | 15 seconds minimum | Convoy hardcoded |
| Smart Min Distance | 10 meters | Varies | Convoy hardcoded |
| GPS Mode | ENABLED | Varies | Convoy hardcoded |

On PROCEED:

- Calls channelViewModel.setConfig(Config(position = ...)) with convoy values

- Android UI minimums bypassed — values written directly to radio firmware

- No reboot triggered

✓ Navigate immediately to Screen 4

## Screen 4 — Channel + PSK (ConvoyChannelConfigScreen)

🔒 GATE: PROCEED button: red + disabled when BT disconnected, green + enabled when connected

Fields displayed (read-only):

- Channel Name — from working config (Process 1/2) or archive (Process 3)

- Encryption Key — AES-256, first 8 chars shown + masked

- Channel Index — 0 (Primary)

- Source — MASTER / RIDE:<eventId> / ARCHIVE

On PROCEED:

- Converts base64 PSK to okio.ByteString

- Calls channelViewModel.setChannels(ChannelSet(...))

⚠  REBOOT TRIGGERED — radio disconnects after channel write

- Button changes to: WAITING FOR RADIO…

- App monitors connectionState for reconnect

- Auto-advances to Screen 5 on reconnect

✓ Navigate to Screen 5 — Verify

## Screen 5 — Verify Config (ConvoyVerifyConfigScreen)

🔒 GATE: Reads radio only when BT connected — waits if disconnected

Auto-reads radio config from ChannelViewModel flows (localConfig + channels) after reconnect. Compares each value against WorkingConfig.

Verification table shows:

- Field name

- Expected value (from working config)

- Actual value (read from radio)

- Pass ✓ or Fail ✗

Fields verified:

- Node Role, Managed Mode

- LoRa Region, Modem Preset, Hop Limit, TX Enabled, TX Power

- Broadcast Interval, GPS Update Interval, Smart Position, Smart Min Interval

- Channel Name, Encryption Key (32-byte presence check)

Result:

✓ All pass: green banner — DONE returns to Convoy map

⚠  Any fail: red banner listing failed fields — DONE available but user sees what failed

# ARCHIVE — Recovery Position

Before any write, the current radio configuration is archived to internal storage. This is the safety net for all write operations.

- Archive path: context.filesDir/convoy_backups/<nodeId>/pre_write_<timestamp>.json

- Archive contains: full DeviceProfile, localConfig, channelSet, hardware info

- Archive is written BEFORE any config changes

- If archive fails: write is blocked — archive is mandatory not optional

- Restore from Archive (Process 3) reads these files to restore radio to prior state

# WORKING CONFIG — Data Structure

WorkingConfig is the single source of truth for all values to be written. Built once before Screen 1, stored in ConvoyViewModel, read by all five screens.

- nodeId — hardware ID of target radio

- longName — node display name

- channelName — convoy channel name

- channelPsk — AES-256 encryption key (base64)

- loraRegion — e.g. US

- loraModemPreset — e.g. LONG_FAST

- loraBandwidth, loraSpreadFactor, loraCodingRate

- loraHopLimit, loraTxEnabled, loraTxPower

- source — MASTER / RIDE:<eventId> / ARCHIVE:<filename>

# SETTINGS WARNING — Convoy Channel Detection

If a user opens the standard Meshtastic Settings screens after a convoy config has been applied, changes may break convoy connectivity. A warning banner is shown on all standard config screens if the primary channel name starts with 'Convoy' (case insensitive).

- Detection: channels.settings.firstOrNull()?.name?.startsWith("Convoy", ignoreCase = true)

- Warning message: '⚠ This radio is configured for Convoy operation. Changes here may break convoy connectivity.'

- Settings remain accessible — warning does not block

- Warning clears if channel name no longer starts with 'Convoy'

# GPS INTERVAL DESIGN

Android's standard Meshtastic config screens enforce minimum interval restrictions that are too conservative for convoy tracking. Our convoy screens bypass these restrictions and write values directly to the firmware.

At 30 MPH (44 feet/second):

- 5-second broadcast = 220 feet between position updates to mesh

- 1-second GPS = 44-foot resolution on KML track recording

- Smart position at 10 meters = broadcasts triggered by movement, not just timer

Live tuning via Convoy Settings slider:

- Broadcast interval slider: 1–60 seconds

- Applies immediately via setConfig(Config(position = ...)) — no reboot

- Used to evaluate mesh congestion in real-world convoy environment

- At Long Fast preset: 10 nodes x 1 broadcast/sec risks channel saturation

- Recommended starting point: 5 seconds, tune down if mesh handles it