CONVOY TRACKER

WorkingConfig  →  Radio Write Function Field Mapping

March 19, 2026  |  47 Verified Fields  +  User/Owner  +  Display/Module  |  feature/convoy-event-ride

CRITICAL RULE: Every write function sends the COMPLETE config group. There is no delta write. Zero fields overwrite radio values.

setConfig(lora) sends all 18 LoRa fields. setConfig(device) sends all 9 Device fields. setConfig(position) sends all 12 Position fields. setChannels() sends all 8 channel fields per index.  WorkingConfig must carry ALL fields for every group written.

Batch strategy: wrap all writes in beginEditSettings → [4 writes] → commitEditSettings to trigger only ONE radio reboot instead of four.

Write Function Summary

| Write Function | Field Count | Kotlin Call in MeshService | Triggers Reboot | Group in 47 |
| --- | --- | --- | --- | --- |
| setConfig(lora) | 18 | buildAdminPacket { setConfig = Config { lora = loraConfig } } | YES — unless batched | Fields 1–18 |
| setConfig(device) | 9 | buildAdminPacket { setConfig = Config { device = deviceConfig } } | YES — unless batched | Fields 19–27 |
| setConfig(position) | 12 | buildAdminPacket { setConfig = Config { position = posConfig } } | YES — unless batched | Fields 28–39 |
| setChannels() | 8 per ch | buildAdminPacket { setChannel = channel } | YES — unless batched | Fields 40–47 |
| setOwner() | 3 | buildAdminPacket { setOwner = user } | NO | Not in 47 fields |
| setConfig(display) | 4 | buildAdminPacket { setConfig = Config { display = dispConfig } } | YES | D1–D4 (master.cfg only) |
| setModuleConfig() | 17 | buildAdminPacket { setModuleConfig = moduleConfig } | YES | M1–M17 (master.cfg only) |

GROUP 1  —  LoRa Config  (Fields 1–18)  →  setConfig(lora)

AdminMessage: buildAdminPacket { setConfig = Config { lora = workingConfig.loraConfig } }

Write trigger: Delta correction after master.cfg import, OR direct LoRa update in 4-piece strategy.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | loraRegion | lora.region | setConfig(lora) | LoRaConfig | MANDATORY. US for convoy. Radio will not transmit if UNSET. |
| 2 | usePreset | lora.use_preset | setConfig(lora) | LoRaConfig | true = use modem_preset enum. false = use BW/SF/CR fields directly. |
| 3 | modemPreset | lora.modem_preset | setConfig(lora) | LoRaConfig | LONG_FAST = default. Only active when use_preset = true. |
| 4 | bandwidth | lora.bandwidth | setConfig(lora) | LoRaConfig | 0 when use_preset=true. Firmware ignores it. Set 0 unless custom preset. |
| 5 | spreadFactor | lora.spread_factor | setConfig(lora) | LoRaConfig | 0 when use_preset=true. Firmware ignores it. Set 0 unless custom preset. |
| 6 | codingRate | lora.coding_rate | setConfig(lora) | LoRaConfig | 0 when use_preset=true. Firmware ignores it. Set 0 unless custom preset. |
| 7 | hopLimit | lora.hop_limit | setConfig(lora) | LoRaConfig | Max 7. Convoy standard = 3. |
| 8 | txPower | lora.tx_power | setConfig(lora) | LoRaConfig | 0 = max legal power for region. Do not override unless testing. |
| 9 | txEnabled | lora.tx_enabled | setConfig(lora) | LoRaConfig | MUST be true. Radio will not transmit if false. |
| 10 | channelNum | lora.channel_num | setConfig(lora) | LoRaConfig | Frequency slot 1–NUM_SLOTS. 0 = older hash-based algorithm. |
| 11 | frequencyOffset | lora.frequency_offset | setConfig(lora) | LoRaConfig | Crystal calibration correction. Usually 0.0. |
| 12 | overrideDutyCycle | lora.override_duty_cycle | setConfig(lora) | LoRaConfig | false = comply with EU duty cycle limits. Keep false. |
| 13 | sx126xRxBoostedGain | lora.sx126x_rx_boosted_gain | setConfig(lora) | LoRaConfig | SX126x chip only. Small power cost for better RX sensitivity. |
| 14 | overrideFrequency | lora.override_frequency | setConfig(lora) | LoRaConfig | 0 = use standard channel slot calculation. |
| 15 | paFanDisabled | lora.pa_fan_disabled | setConfig(lora) | LoRaConfig | Hardware-specific PA fan pin. false normally. |
| 16 | ignoreIncoming | lora.ignore_incoming | setConfig(lora) | LoRaConfig | Deprecated array. Send as empty []. Do not populate. |
| 17 | ignoreMqtt | lora.ignore_mqtt | setConfig(lora) | LoRaConfig | false = process MQTT-relayed packets normally. |
| 18 | configOkToMqtt | lora.config_ok_to_mqtt | setConfig(lora) | LoRaConfig | false = do not consent to MQTT uplink of our packets. |

GROUP 2  —  Device Config  (Fields 19–27)  →  setConfig(device)

AdminMessage: buildAdminPacket { setConfig = Config { device = workingConfig.deviceConfig } }

Write trigger: Part of 4-piece strategy. Convoy sets role=CLIENT, managed=true.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| 19 | deviceRole | device.role | setConfig(device) | DeviceConfig | CLIENT for all rider nodes. TRACKER optional for asset-only nodes. |
| 20 | rebroadcastMode | device.rebroadcast_mode | setConfig(device) | DeviceConfig | ALL = rebroadcast all mesh packets. Standard for convoy nodes. |
| 21 | buttonGpio | device.button_gpio | setConfig(device) | DeviceConfig | 0 = use hardware pin definition. Do not override. |
| 22 | buzzerGpio | device.buzzer_gpio | setConfig(device) | DeviceConfig | 0 = use hardware pin definition. Do not override. |
| 23 | nodeinfoSecs | device.nodeinfo_broadcast_secs | setConfig(device) | DeviceConfig | Seconds between NodeInfo broadcasts. Default 900. |
| 24 | doubleTapAsButton | device.double_tap_as_button_press | setConfig(device) | DeviceConfig | false unless device has supported accelerometer. |
| 25 | isManagedMode | device.is_managed | setConfig(device) | DeviceConfig | true = app manages config, disables manual device changes. |
| 26 | disableTripleClick | device.disable_triple_click | setConfig(device) | DeviceConfig | false = allow triple-click button to toggle GPS on/off. |
| 27 | tzDef | device.tzdef | setConfig(device) | DeviceConfig | POSIX timezone e.g. MST7MDT,M3.2.0,M11.1.0 for Arizona/Mountain. |

GROUP 3  —  Position Config  (Fields 28–39)  →  setConfig(position)

AdminMessage: buildAdminPacket { setConfig = Config { position = workingConfig.positionConfig } }

Write trigger: Part of 4-piece strategy. Forces GPS enabled, frequent broadcast, smart position on.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| 28 | positionBroadcastSecs | position.position_broadcast_secs | setConfig(position) | PositionConfig | Seconds between GPS position broadcasts. Convoy: 30–60s. |
| 29 | positionBroadcastSmartEnabled | position.position_broadcast_smart_enabled | setConfig(position) | PositionConfig | true = broadcast only when position changes by min distance. |
| 30 | fixedPosition | position.fixed_position | setConfig(position) | PositionConfig | false for all moving convoy vehicles. |
| 31 | gpsEnabled | position.gps_enabled | setConfig(position) | PositionConfig | true = use GPS hardware. MANDATORY for convoy tracking. |
| 32 | gpsUpdateInterval | position.gps_update_interval | setConfig(position) | PositionConfig | Seconds between GPS hardware polls. Usually 30. |
| 33 | gpsAttemptTime | position.gps_attempt_time | setConfig(position) | PositionConfig | Seconds to wait for GPS fix before giving up. |
| 34 | positionFlags | position.position_flags | setConfig(position) | PositionConfig | Bitmask controlling which fields are in position packets (speed, heading, altitude, etc). |
| 35 | rxGpio | position.rx_gpio | setConfig(position) | PositionConfig | 0 = hardware default GPS serial RX pin. |
| 36 | txGpio | position.tx_gpio | setConfig(position) | PositionConfig | 0 = hardware default GPS serial TX pin. |
| 37 | broadcastSmartMinInterval | position.broadcast_smart_minimum_interarrival_secs | setConfig(position) | PositionConfig | Minimum seconds allowed between smart position broadcasts. |
| 38 | broadcastSmartMinDistance | position.broadcast_smart_minimum_distance | setConfig(position) | PositionConfig | Minimum meters of movement required to trigger smart broadcast. |
| 39 | gpsMode | position.gps_mode | setConfig(position) | PositionConfig | ENABLED / DISABLED / NOT_PRESENT. Convoy: ENABLED. |

GROUP 4  —  Channel Config  (Fields 40–47)  →  setChannels()

AdminMessage: buildAdminPacket { setChannel = Channel { index=0, role=PRIMARY, settings=channelSettings } }

Write trigger: MANDATORY for every ride — channel name and PSK are ride-specific. Also part of 4-piece strategy.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| 40 | channelIndex | (Channel.index) | setChannels() | Channel | Always 0 for primary. Set on the Channel wrapper object, not inside ChannelSettings. |
| 41 | channelRole | (Channel.role) | setChannels() | Channel | PRIMARY (1). Set on Channel wrapper, not inside ChannelSettings. |
| 42 | channelName | settings.name | setChannels() | ChannelSettings | CONVOY-XXXX for ride channels. < 12 bytes. MANDATORY — ride-specific. |
| 43 | channelPsk | settings.psk | setChannels() | ChannelSettings | 32-byte AES256 key from SecureRandom. MANDATORY — ride-specific. Must match all riders. |
| 44 | uplinkEnabled | settings.uplink_enabled | setChannels() | ChannelSettings | false for off-grid convoy. No MQTT uplink. |
| 45 | downlinkEnabled | settings.downlink_enabled | setChannels() | ChannelSettings | false for off-grid convoy. No MQTT downlink. |
| 46 | positionPrecision | settings.module_settings.position_precision | setChannels() | ModuleSettings | 32 = full precision GPS in packets. 0 = never send position over this channel. |
| 47 | isMuted | settings.module_settings.is_muted | setChannels() | ModuleSettings | false = allow external notifications when messages arrive on this channel. |

USER / OWNER  —  setOwner()  (NOT in the 47 fields — separate write path)

short_name is STRIPPED from master.cfg before import so riders keep their own callsign. long_name IS written by master.cfg import. Neither is verifiable by read-back.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| — | longName | user.long_name | setOwner() | User | Written by master.cfg import. CANNOT be read back by Verify screen. |
| — | shortName | user.short_name | setOwner() | User | STRIPPED from master.cfg before import so riders keep own callsign. |
| — | isLicensed | user.is_licensed | setOwner() | User | Ham radio license flag. Not written by convoy flow. |

DISPLAY (4) + MODULE (17)  —  master.cfg import ONLY  —  NOT in 4-piece write flow

These fields ARE verified by your Verify screen but are NOT individually written by setConfig() in the convoy flow. They are written atomically by InstallProfileUseCase (master.cfg binary import) and are not touched again by the 4-piece strategy.

OPEN BUG: Verify screen compares Module fields against 0/false defaults. Fix: read moduleConfig from radio and populate WorkingConfig.moduleFields before verify runs.

| # | WorkingConfig Field | master_config.json Key | Write Function | Protobuf Group | Notes |
| --- | --- | --- | --- | --- | --- |
| D1 | displayUnits | display.units | master.cfg import only | DisplayConfig | setConfig(display) not in 4-piece flow. Written atomically by InstallProfileUseCase. |
| D2 | screenTimeout | display.screen_on_secs | master.cfg import only | DisplayConfig | setConfig(display) not in 4-piece flow. Written atomically by InstallProfileUseCase. |
| D3 | autoBrightness | display.auto_screen_carousel_secs | master.cfg import only | DisplayConfig | setConfig(display) not in 4-piece flow. Written atomically by InstallProfileUseCase. |
| D4 | compassNorthTop | display.compass_north_top | master.cfg import only | DisplayConfig | setConfig(display) not in 4-piece flow. Written atomically by InstallProfileUseCase. |
| M1–M17 | moduleFields[17] | module_config.* | master.cfg import only | ModuleConfig | All 17 module fields. Telemetry, MQTT, Serial, Ext Notification, Range Test, Store Forward, Neighbor Info, Detection Sensor, Audio. VERIFY BUG: currently compare against 0/false defaults. |

Recommended 4-Piece Transaction Sequence  (1 reboot)

// Step 1: Open transaction — radio buffers all changes

sendToRadio(buildAdminPacket { beginEditSettings = true })

// Step 2: Write all 4 groups

sendToRadio(buildAdminPacket { setConfig = Config { lora     = workingConfig.loraConfig     } })

sendToRadio(buildAdminPacket { setConfig = Config { device   = workingConfig.deviceConfig   } })

sendToRadio(buildAdminPacket { setConfig = Config { position = workingConfig.positionConfig } })

sendToRadio(buildAdminPacket { setChannel = workingConfig.buildChannelProto() })

// Step 3: Commit — triggers ONE reboot

sendToRadio(buildAdminPacket { commitEditSettings = true })

// Step 4: Navigate to ReconnectWait screen → then Verify screen

IMPORTANT: After master.cfg import (InstallProfileUseCase), the radio has already rebooted and all 47 fields including Display and Module are set. The 4-piece transaction is used ONLY for delta corrections — in most cases only setChannels() fires for the ride-specific name and PSK. Use WorkingConfig to compare against master_config.json and only include groups that actually differ.

CONVOY TRACKER  |  WorkingConfig Write Mapping  |  47 Fields + User + Display + Module  |  March 19, 2026