GroupTrack

V2.4 Open Tasks + V2.5 Planned Tasks

Updated April 17, 2026 — Includes field test findings

# 1. V2.4 Open Tasks

| ID | Issue | Priority | Fix Approach |
| --- | --- | --- | --- |
| BLE-01 | BLE scan button missing — ApplicationRegistrationFailed on resume | P1 | Reset GATT registration on onResume/onPause lifecycle |
| BLE-02 | Radio requires power cycle in some scenarios | P1 | Same fix as BLE-01 |
| LEAD-01 | Lead cart assignment unreliable — distance accumulator | P1 | Replace with manual declaration packet — full redesign. See Section 2. |
| LEAD-02 | Lead cart displays behind other carts on map | P1 | Investigate computeSortPositions() heading sort. Lead always position 1 when locked. |
| TILES-01 | Partial layer downloads — only SAT downloading | P1 | Root cause: Thunderforest TRAIL quota likely exceeded (150k tiles/month free tier). Investigate per-source failure reporting. Consider removing Thunderforest entirely in favor of Utah UGRC GeoJSON trail overlay. |
| ANDROID16-01 | RECORD crash on Android 16 | FIXED aad4dc0d5 | ACCESS_BACKGROUND_LOCATION + runtime check added |
| BUILD-01 | Debug APK installed on tester devices | P1 | Enforce: V2.4 = assembleGoogleRelease only. copyApkToDownloads = V3 debug only. |

# 2. Lead Cart Redesign — Last V2.4 Release

Full redesign. Remove all distance accumulator logic. Replace with PRIVATE_APP declaration packet.

## 2.1  RECORD Button Decision Tree

- RECORD tapped

- No radio connected -> auto-assign My Cart as lead, use device GPS, start recording (standalone mode)

- Radio connected + lead already locked -> start recording immediately, no dialog

- Radio connected + no lead locked -> show dialog: I AM LEAD / SELECT LEAD / NO LEAD YET

- I AM LEAD -> broadcast PRIVATE_APP packet with my node ID, lock lead on all carts, start recording

- SELECT LEAD -> show node list, tap to select, broadcast packet with selected node ID, lock, start recording

- NO LEAD YET -> start recording, lead unassigned, assign later via RECALC LEAD in settings

## 2.2  Code to Remove

- nodeDistanceAccum map and all accumulator logic in ConvoyViewModel.tick()

- nodeLastLat / nodeLastLon accumulator maps

- lockedLeadNodeId distance-based election logic

- _leadLockedFlag distance trigger and LOCK FIRED log

- LEAD_LOCK_DISTANCE_MILES in ConvoyConfig

- Lead lock distance slider in ConvoySettingsScreen

- recalcLead() distance implementation — replace with dialog launch

## 2.3  Declaration Packet Spec

| Field | Bytes | Value |
| --- | --- | --- |
| packet_type | 1 | 0x02 = lead declaration |
| lead_node_id | 4 | Meshtastic node ID of designated lead cart |
| declared_by | 4 | Node ID of the Android device sending the declaration |
| timestamp | 4 | Unix seconds |
| TOTAL | 13 | portNum 256 PRIVATE_APP — well within 240-byte limit |

# 3. V2.5 Map Discussion — Monday

## 3.1  Tile Source Strategy — Thunderforest Decision

OPEN ISSUE: Only SAT layer downloading. Root cause suspected: Thunderforest TRAIL free tier quota (150,000 tiles/month) likely exhausted after 6+ area downloads. Tiles return 403/429, downloader logs failure and continues with other sources.

RECOMMENDATION: Remove Thunderforest TRAIL source entirely. Replace with Utah UGRC GeoJSON trail overlay (dynamic bounding box query — no tile quota). This provides better trail data for St. George/southern Utah riding area with no quota limits.

| Source | Current | Recommendation | Reason |
| --- | --- | --- | --- |
| SAT | Esri World Imagery — KEEP | Keep | Primary riding view. No quota. Working. |
| TOPO | Esri World Topo — KEEP | Keep | Planning view. No quota. Working. |
| TOPO+ | OpenTopoMap — KEEP | Keep | Dark trails. No quota. Working. |
| TRAIL | Thunderforest Outdoors — REMOVE | Remove + replace with UGRC GeoJSON overlay | 150k/month quota exhausted. Utah UGRC dynamic overlay has no quota and better local data. |
| HYB | Esri Hybrid — KEEP | Keep | Satellite + labels. No quota. Working. |

## 3.2  Utah UGRC Trail Overlay (MAP-04)

- Source: Utah SGID Trails and Pathways FeatureServer

- Endpoint: https://services1.arcgis.com/99lidPhWCzftIe9K/ArcGIS/rest/services/TrailsAndPathways/FeatureServer/0

- Query type: Dynamic bounding box on map pan — no full download, no quota

- OHV filter: CartoCode=4 (road-concurrent forest service roads, double track)

- Tap behavior: Show trail name and allowed use on tap

- Best base: SAT or HYB — trails visible at Z16+

- No API key required — public SGID data

## 3.3  Partial Download Fix

- Add per-source download status to download complete screen

- Show which sources succeeded and which failed with error code

- Allow re-download of failed sources without re-downloading successful ones

- Log Thunderforest HTTP response code to diagnose quota vs network issue

## 3.4  Map Functions — Priority Order for V2.5

| ID | Function | Priority | Notes |
| --- | --- | --- | --- |
| MAP-01 | GPS recording interval 1s | P1 | ConvoyGpsService.kt:249 — 3000L to 1000L |
| MAP-02 | Lead cart 5-point broadcast | P1 | Same PRIVATE_APP infrastructure as lead declaration packet |
| MAP-03 | KML/GPX route overlay | P1 | Leaflet omnivore. Android file picker. Bridge to WebView. |
| MAP-04 | Utah UGRC trail overlay | P1 | Replaces Thunderforest. Dynamic bbox. No quota. Better local data. |
| MAP-05 | My Tracks overlay on convoy map | P1 | Display recorded GPX tracks as overlay |
| MAP-06 | USFS national OHV trail overlay | P2 | All 50 states NF land — supplement to UGRC for non-Utah rides |
| MAP-07 | Waypoint management | P2 | Drop pins, name, persist, show on map |
| MAP-08 | Tile download manager — per-source status | P2 | Show success/fail per source. Re-download failed only. |
| MAP-09 | Post-ride snap-to-trail | P3 | Phase C — requires MAP-01 and MAP-02 |

# 4. V2.5 Other Planned Tasks

## 4.1  Package Migration

- New package: com.grouptrack.android

- Tile path: /sdcard/GroupTrack/tiles/ — shared external, survives package changes

- Migration: single renameTo() on tiles/ directory on first launch

- Google Play listing under GroupTrack brand

## 4.2  Other Open Items

- BLE-01/02 — stale GATT registration fix

- GPS-01/02 — standalone GPS recording without radio

- Remove Live/Demo mode toggle from V3

- TILES-01 — remove Thunderforest, add UGRC overlay

GroupTrack  |  GroupTrack_V24_OpenTasks_V25_Plan_v2  |  April 17, 2026