GroupTrack V2.4 — Radio & Map Revision Items

Date: April 13, 2026  |  Branch: feature/convoy-event-ride

Purpose: All V2.4 radio and map improvement items with implementation sequence

Implementation Sequence — In Order

Build and test each step before proceeding to the next.

| Step | Item | Effort | Impact |
| --- | --- | --- | --- |
| 1 | MAP-01 — GPS recording 3s → 1s | 1 line | Better local track all carts |
| 2 | MAP-02 — Lead cart multi-point broadcast | Medium | Smooth 1-second lead track all devices |
| 3 | MAP-03 — KML/GPX overlay on convoy map | Medium | Rider displays pre-loaded route |
| 4 | MAP-04 — Utah UGRC trail overlay | Medium | Live trail lines on SAT/TOPO+ base |
| 5 | MAP-05 — USFS national OHV trail overlay | Medium | All-states trail coverage |
| 6 | MAP-06 — Post-ride snap-to-trail | Large | Clean track export for sharing |
| 7 | RADIO-01 — SHORT_FAST bandwidth analysis | Research | Evaluate faster update intervals |

MAP-01 — GPS Recording Interval

File: ConvoyGpsService.kt  |  Line: 249

| Field | Detail |
| --- | --- |
| Current value | 3000L (3 seconds minimum interval |
| Target value | 1000L (1 second minimum interval |
| Change | lm.requestLocationUpdates(provider, 1000L, 0f, listener) |
| Effect | All carts record 1-second GPX tracks locally |
| Battery impact | Minimal on modern Android |
| Risk | Low — Android may deliver less frequently depending on GPS hardware |
| Test | Pull GPX from device after ride — confirm 1-second timestamps |

MAP-02 — Lead Cart Multi-Point Broadcast

Buffer 5 GPS points on lead cart over 5-second window. Broadcast as single Meshtastic PRIVATE_APP packet. Zero channel traffic change.

Packet Specification

| Field | Bytes | Notes |
| --- | --- | --- |
| packet_type | 1 | 0x01 = lead cart multi-point track |
| node_id | 4 | Lead cart Meshtastic node ID |
| point_count | 1 | Always 5 — one per second over 5-second window |
| lat[0..4] | 20 | Float32 x5 — latitude per point |
| lon[0..4] | 20 | Float32 x5 — longitude per point |
| timestamp[0..4] | 20 | Unix seconds x5 — date+time stamp per point |
| heading[0..4] | 10 | UInt16 x5 — degrees 0-359 per point |
| speed[0..4] | 10 | UInt16 x5 — mph x10 (e.g. 125 = 12.5 mph) |
| TOTAL | 86 | Well within 240-byte Meshtastic limit |

Why Heading and Speed Matter

•  Heading enables correct interpolation between points on curves and switchbacks

•  Speed confirms cart is moving — filters GPS drift when stationary

•  Together produce a track that follows the actual route rather than straight-line approximation

•  Speed data enables real-time convoy spread calculation with better accuracy

Timestamp Requirement — Critical

Every point MUST include a Unix timestamp. This enables:

•  Correct ordering of received points even if radio packets arrive out of order

•  Accurate speed calculation between consecutive points

•  Post-ride track reconstruction with correct timing

•  Snap-to-trail alignment using timestamp for interpolation

Implementation

| Field | Detail |
| --- | --- |
| Meshtastic portNum | 256 (PRIVATE_APP) — no firmware changes required |
| New file | ConvoyLeadTrackBroadcaster.kt — buffer, pack, and send |
| Modified file | ConvoyEngine.kt — parse multi-point packets, update lead track |
| Modified file | convoy_map.html — render 5 positions per 5-second interval |
| Channel utilization | Unchanged — 5 carts x 1 packet/5 seconds = 25% |
| Trigger | Lead cart only — when isLead == true in ConvoyEngine |

MAP-03 — KML/GPX Overlay on Convoy Map

Rider imports a KML or GPX route file and displays it as an overlay on the convoy map.

| Field | Detail |
| --- | --- |
| Library | Leaflet omnivore plugin — handles KML and GPX natively |
| Entry point | Android file picker — select from device storage |
| Bridge | Pass file content from Android to Leaflet via evaluateJavascript |
| Display | Route line rendered over SAT/TOPO/TRAIL/TOPO+ tile base |
| Toggle | Show/hide overlay without removing |
| Multiple overlays | Stack multiple KML/GPX files on same map |
| Persist | Named overlays persist across app sessions |

MAP-04 — Utah UGRC Trail Overlay

Dynamic GeoJSON trail overlay from Utah SGID — queried by bounding box as rider pans map.

| Field | Detail |
| --- | --- |
| Source | Utah SGID Trails and Pathways FeatureServer |
| Endpoint | https://services1.arcgis.com/99lidPhWCzftIe9K/ArcGIS/rest/services/TrailsAndPathways/FeatureServer/0 |
| Query type | Dynamic bounding box on map pan — no full download |
| OHV filter | CartoCode=4 (Road-concurrent) — forest service roads, double track |
| St. George bbox | West:-114.1 South:36.85 East:-113.3 North:37.5 |
| Tap behavior | Show trail name and allowed use on tap |
| Best tile base | SAT or TOPO+ — trails visible at Z16+ |

MAP-05 — USFS National OHV Trail Overlay

National Forest OHV trails for all 50 states — same dynamic bounding box query approach.

| Field | Detail |
| --- | --- |
| Source | USFS National Forest System Trails Feature Layer |
| Endpoint | https://data-usfs.hub.arcgis.com/datasets/usfs::national-forest-system-trails-feature-layer |
| OHV filter | ALLOWED_TERRA_USE LIKE '%4%' OR '%5%' OR '%6%' |
| Field meaning | 4=motorcycle 5=ATV 6=full-size 4WD |
| Coverage | All 50 states — National Forest land only |
| Supplement | Utah UGRC provides better local detail within Utah |

MAP-06 — Post-Ride Track Snap-to-Trail

Phase C. After ride completes — snap the recorded GPX to nearest trail line for clean export.

| Field | Detail |
| --- | --- |
| When | Post-ride only — not real-time |
| Input | Device local GPX track at 1-second resolution (requires MAP-01) |
| Reference | Utah UGRC or USFS trail GeoJSON for the ride area |
| Algorithm | Nearest-neighbor projection of each point onto closest trail segment |
| Heading use | Heading field narrows candidate segments — eliminates wrong-direction matches |
| Output | Clean GPX for Gaia/OnX/Garmin/Strava/AllTrails sharing |
| Dependency | Requires MAP-01 and MAP-02 for acceptable accuracy |
| Phase | Phase C — after MAP-01 through MAP-05 complete |

RADIO-01 — SHORT_FAST Bandwidth Analysis

Research item. Evaluate whether switching from LONG_FAST to SHORT_FAST enables faster update intervals with acceptable range trade-off.

Current — LONG_FAST

| Parameter | Value |
| --- | --- |
| Modulation | LoRa SF11 BW250 |
| Approx time on air per packet | ~250ms |
| 5 carts at 5-second interval | 25% channel utilization |
| Typical range | 3-5 miles line of sight |
| Max safe packet rate | 1 packet/second (25% of 4 packets/second capacity |

SHORT_FAST Analysis

| Parameter | Value |
| --- | --- |
| Modulation | LoRa SF7 BW500 |
| Approx time on air per packet | ~30ms — approximately 8x faster than LONG_FAST |
| Theoretical packet capacity | ~30 packets/second |
| 5 carts at 1-second interval | ~17% channel utilization — safe |
| 5 carts at 2-second interval | ~8% channel utilization — very safe |
| Range trade-off | Significantly shorter — approx 0.5-1 mile line of sight |
| Range verdict | TOO SHORT for convoy use — riders can separate beyond range |

MEDIUM_FAST — Better Trade-off

| Parameter | Value |
| --- | --- |
| Modulation | LoRa SF9 BW500 (approx |
| Time on air per packet | ~75ms — approx 3x faster than LONG_FAST |
| 5 carts at 2-second interval | ~19% channel utilization — safe |
| Range | 1.5-2.5 miles line of sight — marginal for convoy |
| Verdict | Borderline — depends on terrain and convoy spread |

Recommendation

Stay on LONG_FAST. The multi-point broadcast in MAP-02 solves the track quality problem without any modem change. Range is more important than update frequency for OHV convoy use where riders can separate by 1-2 miles on technical terrain.

•  LONG_FAST 5-second interval with 5-point payload = best of both worlds

•  1-second effective track resolution on all devices

•  Full 3-5 mile range maintained

•  25% channel utilization — safe headroom

Future consideration: if convoy moves to urban/close-proximity use where range is not a concern, SHORT_FAST at 1-2 second intervals becomes viable. Document as future option.

Key Files Reference

| File | Item |
| --- | --- |
| ConvoyGpsService.kt line 249 | MAP-01 — change 3000L to 1000L |
| ConvoyLeadTrackBroadcaster.kt (new) | MAP-02 — buffer 5 points, pack, broadcast |
| ConvoyEngine.kt | MAP-02 — parse multi-point packets, update lead track |
| convoy_map.html | MAP-02/03/04/05 — Leaflet overlay rendering |
| ConvoyScreen.kt | MAP-03 — file picker and WebView bridge |
| ConvoyConfig.kt | TRACK_EXPORT_FORMAT GPX/KML setting |