GroupTrack

Field Test Checklist — April 4, 2026  v1

Two radios + two Androids  |  Branch: feature/convoy-event-ride  |  Commit: 649d350f6

# Pre-Test Setup

| ☐ | Task | Device | Status |
| --- | --- | --- | --- |
| ☐ | Force stop app on both Androids before launch | Both | OPEN |
| ☐ | Delete tiles on Android 1 — tiles moved to external storage, need re-download | Android 1 | OPEN |
| ☐ | Download Gooseberry Mesa tile area on Android 1 before leaving | Android 1 | OPEN |
| ☐ | Verify both radios on Convoy channel — channel 5, correct PSK | Both radios | OPEN |
| ☐ | Check lead lock slider in Convoy Settings — verify default 330 feet | Both | OPEN |

# FT-01 — Lead Lock Algorithm

Press RECORD on both devices. Drive. Verify correct node locks at ~330 feet (~11 sec at 20mph).

| ☐ | ID | Test | Est. | Result |
| --- | --- | --- | --- | --- |
| ☐ | FT-01a | Press RECORD. Drive. Toast fires 'XXXX Locked as Lead' at ~330 feet. | 5 min | OPEN |
| ☐ | FT-01b | Correct node locked — furthest traveled wins, not stationary cart. | verify | OPEN |
| ☐ | FT-01c | Pull debug log — verify LOCK FIRED on correct nodeId, accumulator values correct. | 5 min | OPEN |
| ☐ | FT-01d | RECALC LEAD — lock clears, re-election opens, new lock fires after 330 feet. | 5 min | OPEN |
| ☐ | FT-01e | Adjust slider to 600 feet. RECALC. Verify new lock fires at longer distance. | 10 min | OPEN |

# FT-02 — Lead Track (Android 1)

Verify no track draws before lock. Clean lead track after lock. No false lines to stationary carts.

| ☐ | ID | Test | Est. | Result |
| --- | --- | --- | --- | --- |
| ☐ | FT-02a | LEAD ONLY mode — no track lines draw before lock fires. | verify | OPEN |
| ☐ | FT-02b | After lock — clean black track follows locked lead node only. | verify | OPEN |
| ☐ | FT-02c | No false lines crossing field to stationary cart. | verify | OPEN |
| ☐ | FT-02d | Switch to MULTI TRACK — each node draws its own trail in its own color. | 5 min | OPEN |
| ☐ | FT-02e | MULTI TRACK colors match node marker colors on map. | verify | OPEN |

# FT-03 — Android 2 (No GPS Hardware)

Android 2 uses radio GPS fallback. Verify RECORD works and track draws from radio position.

| ☐ | ID | Test | Est. | Result |
| --- | --- | --- | --- | --- |
| ☐ | FT-03a | Press RECORD on Android 2 — no crash. Service starts cleanly. | verify | OPEN |
| ☐ | FT-03b | Lead lock fires on Android 2 — same node as Android 1. | verify | OPEN |
| ☐ | FT-03c | No spider web of lines on Android 2 map — pre-lock suppression working. | verify | OPEN |
| ☐ | FT-03d | Track records via radio GPS — GPX file written to external storage. | verify | OPEN |

# FT-04 — Tile Storage

Tiles moved from filesDir to external storage. Verify tiles survive app update and render correctly.

| ☐ | ID | Test | Est. | Result |
| --- | --- | --- | --- | --- |
| ☐ | FT-04a | Download Gooseberry Mesa area. Switch to offline. Tiles render at z18. | 10 min | OPEN |
| ☐ | FT-04b | Install updated APK. Launch. Verify tiles still present — not wiped by install. | 5 min | OPEN |
| ☐ | FT-04c | HIDE DOWNLOADED toggle still works correctly. | verify | OPEN |

# FT-05 — Event/Ride Services Nav Icon

New Services icon in Meshtastic nav bar. Tap shows Online/Offline chooser.

| ☐ | ID | Test | Est. | Result |
| --- | --- | --- | --- | --- |
| ☐ | FT-05a | Services icon visible in nav bar next to GroupTrack. | verify | OPEN |
| ☐ | FT-05b | Tap Services — Online/Offline chooser bottom sheet appears. | verify | OPEN |
| ☐ | FT-05c | Tap ONLINE — navigates to Sign In screen. | verify | OPEN |
| ☐ | FT-05d | Tap OFFLINE — navigates to Field Radio screen. | verify | OPEN |
| ☐ | FT-05e | Dev toggle on Sign In screen — flip subscribed ON/OFF, verify label changes in chooser. | verify | OPEN |

# Post-Test Commands

Pull debug logs after ride:

adb -s 8624SBCEDF00001789 shell run-as com.geeksville.mesh.google.debug cat files/convoy_debug.log

adb -s 24039703201775 shell run-as com.geeksville.mesh.google.debug cat files/convoy_debug.log

GroupTrack  |  Field Test Checklist v1  |  April 4, 2026