GroupTrack

User Manual

Version 2.4 Production

April 5, 2026  |  Build 20260405.xxxx  |  For GroupTrack Beta Testers

GroupTrack is an off-grid group awareness application for motorcycles, SxS/UTV, mountain bikes, hikers, and overlanders. It provides real-time convoy tracking over Meshtastic LoRa mesh radio — no cell service or internet required during operation.

# Function Index

Use this index to find the page for any GroupTrack function. Testers use this index to locate functions in the certification checklist.

| Function | Section | Page |
| --- | --- | --- |
| App launch and connection | 1 | 2 |
| Convoy map — basic navigation | 2 | 2 |
| RECORD — start/stop GPS track recording | 3 | 3 |
| Lead lock — automatic lead detection | 4 | 3 |
| Lead lock slider — distance threshold | 4.1 | 4 |
| RECALC LEAD — reassign lead mid-ride | 4.2 | 4 |
| I AM LEAD — manual lead assignment | 4.3 | 4 |
| LEAD ONLY / MULTI TRACK toggle | 5 | 5 |
| Node markers and status display | 6 | 5 |
| Auto-pan — group centering | 6.1 | 6 |
| Map layer selection — SAT / HYB / TOPO / RD | 7 | 6 |
| Offline mode toggle | 7.1 | 6 |
| SHOW DOWNLOADED — tile coverage overlay | 7.2 | 7 |
| FIND AREA — geocoder search | 8 | 7 |
| DOWNLOAD REGION — offline tile download | 8.1 | 8 |
| GPX/KML track export toggle | 9 | 9 |
| GEAR panel — settings and build info | 10 | 9 |
| GroupTrack menu — EVENT / RIDE submenu | 11 | 10 |
| Apply Master Config to radio | 12 | 10 |
| Apply Ride Config to radio | 12.1 | 11 |
| Capture Master Config from radio | 13 | 11 |
| Radio config archive and restore | 14 | 12 |
| Build stamp — version identification | 15 | 12 |
| Radio firmware 2.7 — manual channel entry workaround | 16 | 13 |

# 1. App Launch and Radio Connection

GroupTrack requires a Meshtastic-compatible LoRa radio connected via Bluetooth to function. The convoy map is always active — online services (V3.0) require internet connectivity.

| Step | Action |
| --- | --- |
| 1. Launch app | Tap GroupTrack icon. Convoy map loads immediately. |
| 2. Connect radio | Meshtastic → Connection tab → select your radio via Bluetooth. |
| 3. Confirm connection | Radio appears in Nodes tab. GPS coordinates visible when radio has a fix. |
| 4. Return to map | Tap GroupTrack in bottom navigation. Convoy map shows your node. |

✅ Android 2 (no GPS hardware) uses the connected radio's GPS. Allow 2-3 minutes after radio connection for GPS fix to acquire.

# 2. Convoy Map — Basic Navigation

The convoy map is the primary GroupTrack screen. It displays all active nodes, GPS tracks, lead/tail indicators, and map layer controls.

| Element | Description |
| --- | --- |
| Node markers | Colored circle per active node. Color is unique per node — consistent across sessions. |
| Lead indicator | Lead node marker has a crown icon. Track drawn in black. |
| Tail indicator | Rearmost node marker. Dynamic — updates every tick. |
| GPS tracks | Trail drawn behind each node showing path traveled. |
| Auto-pan | Map keeps the group centered automatically during recording. |
| Pinch to zoom | Standard Android gesture. Map stays at zoom level selected. |

# 3. RECORD — GPS Track Recording

Tap the red RECORD button (top left of map) to start recording. All active nodes begin accumulating distance for lead lock election.

| State | Description |
| --- | --- |
| RECORD (red dot) | Recording active. Distance accumulation running. Lead lock election in progress. |
| STOP (grey dot) | Recording stopped. Tracks frozen. Lead lock cleared. |
| Track export | On STOP, track exported as GPX or KML depending on export toggle setting. |

⚠ Recording must be active for lead lock to function. Press RECORD before departure.

# 4. Lead Lock — Automatic Lead Detection

GroupTrack automatically detects the lead vehicle using a distance accumulator. Every active node accumulates distance traveled. The first node to exceed the lead lock threshold is locked as lead for the remainder of the ride.

| Element | Description |
| --- | --- |
| Distance accumulator | Each node tracks total distance traveled since RECORD was pressed. GPS jump filter: movements > 0.5 miles ignored as bad fix. |
| Lock threshold | Default 330 feet (0.0625 miles). Adjustable via slider. First node to exceed threshold wins lead. |
| Lock event | Toast notification: [Node Name] Locked as Lead. Crown icon appears on lead marker. |
| Lock persistence | Lead locked for remainder of ride. Does not change with position — only RECALC changes lead. |

✅ Position your designated lead rider at the front before pressing RECORD. The lead lock algorithm rewards the rider who travels furthest first.

## 4.1 Lead Lock Slider — Distance Threshold

The lead lock slider in the GEAR panel controls the distance threshold before lead is locked. Longer threshold = more time before lock fires.

| Setting | Distance | Best For |
| --- | --- | --- |
| Minimum | 300 ft | Short test rides. Quick lock. Staging area departure. |
| Default | 330 ft | Standard group rides. Recommended setting. |
| Maximum | 1,500 ft | Long rides. Allows more time for group to sort before locking. |

## 4.2 RECALC LEAD — Reassign Lead Mid-Ride

Tap RECALC LEAD to immediately reassign lead to whoever has traveled the furthest total distance. No waiting — lead locks instantly to current furthest node.

| Behavior | Detail |
| --- | --- |
| Immediate lock | Whoever has the highest distance accumulator at the moment RECALC is tapped becomes the new lead instantly. |
| No gap in track | Track recording continues uninterrupted. No break in GPS trail. |
| Use case | Lead rider stops for fuel. Different rider takes point. Tap RECALC to update lead. |

⚠ V2.4 fix: RECALC now immediately locks the furthest node. Prior to this fix, RECALC cleared accumulators and required new distance to be traveled before re-locking.

## 4.3 I AM LEAD — Manual Lead Assignment

Tap I AM LEAD to immediately lock your own node as lead without waiting for the distance accumulator.

✅ Use I AM LEAD when the lead rider is known before departure and you want to skip the election process entirely.

# 5. LEAD ONLY / MULTI TRACK Toggle

Toggle between two track display modes using the button in the map controls.

| Mode | Description |
| --- | --- |
| LEAD ONLY | Only the locked lead node's track is drawn. Black trail. Clean display for navigation focus. |
| MULTI TRACK | Every active node draws its own trail in its unique marker color. Full group visibility. |

# 6. Node Markers and Status Display

Each active node on the mesh appears as a colored circle marker on the convoy map.

| Marker Element | Description |
| --- | --- |
| Color | Unique per node. Consistent across sessions for the same node ID. Used for MULTI TRACK trails. |
| Crown icon | Locked lead node. Appears after lead lock fires. |
| Callsign label | Node short name displayed below marker. |
| Tail marker | Rearmost node. Updates dynamically every tick. |
| My Cart | Your own node. Highlighted differently from remote nodes. |

## 6.1 Auto-Pan — Group Centering

During active recording the map automatically pans to keep the group centered. Auto-pan is disabled when the user manually pans the map and re-engages after 10 seconds of no touch input.

# 7. Map Layer Selection

Tap the layer buttons in the map controls to switch between map sources.

| Button | Source | Description |
| --- | --- | --- |
| SAT | ArcGIS World Imagery | Satellite imagery. Best for trail navigation. Requires internet in online mode. |
| HYB | ArcGIS Hybrid | Satellite + road labels overlay. Requires internet in online mode. |
| TOPO | ArcGIS Topo | Topographic map. Elevation contours. Requires internet. |
| RD | CartoCDN Road | Road map. Minimal data. Works on slow connections. |

## 7.1 Offline Mode Toggle

The OFFLINE toggle switches the map tile source from online (ArcGIS servers) to locally downloaded tiles.

| State | Behavior |
| --- | --- |
| ONLINE | Tiles fetched from ArcGIS servers. Requires active internet. Full zoom range. |
| OFFLINE | Tiles served from local storage. No internet required. Only downloaded zoom levels available. |

⚠ Switch to OFFLINE before entering areas without cell service. Tiles must be downloaded in advance. See Section 8.1 for download instructions.

⚠ Known issue V2.4: HYB layer may appear blank on first selection in offline mode. Workaround: select SAT first, then switch to HYB. Fix targeted for V2.4.2.

## 7.2 SHOW DOWNLOADED — Tile Coverage Overlay

Tap SHOW DOWNLOADED to display a blue bounding rectangle showing the extent of downloaded tiles on disk. The rectangle is calculated from actual tile files — not from the original download selection.

| Behavior | Detail |
| --- | --- |
| Blue rectangle | Shows the geographic extent of all Z18 tiles present on the device. Accurate at all zoom levels. |
| Toggle off | Tap HIDE DOWNLOADED to remove the overlay. |
| Use before ride | Confirm coverage of your planned riding area before departure. If your route extends beyond the blue box, download additional tiles. |

✅ Always verify downloaded coverage before a ride in areas without cell service. The blue box shows exactly what you have.

# 8. FIND AREA — Geocoder Search

Tap FIND AREA to search for a location by name. The map flies to the result and centers on it for offline tile download planning.

| Step | Action |
| --- | --- |
| 1. Tap FIND AREA | Search bar appears at top of map. |
| 2. Enter location | Type city, park, trail area. Example: Gooseberry Mesa UT, Sand Hollow UT, Moab Utah. |
| 3. Tap FIND | Geocoder searches. Returns up to 5 results. |
| 4. Select result | Tap the correct result. Map flies to center. Blue pin marks location. |
| 5. Plan download | Zoom and explore. Tap DOWNLOAD REGION to select tiles. |

✅ Add state abbreviation for best results: 'Sand Hollow UT' not just 'Sand Hollow'. Raw coordinates also accepted: 37.44, -113.25

## 8.1 DOWNLOAD REGION — Offline Tile Download

Tap DOWNLOAD REGION to draw a rectangle on the map and download all tiles within that area for offline use. Tiles are stored at zoom levels 10-18.

| Step | Detail |
| --- | --- |
| 1. Tap DOWNLOAD REGION | Rectangle draw tool activates on map. |
| 2. Draw rectangle | Touch and drag to select your ride area. Include staging area and all planned trails. |
| 3. Review estimate | Dialog shows tile count, MB estimate, and source layer. |
| 4. Tap DOWNLOAD | Download begins. Keep app open. Toast confirms completion. |
| 5. Verify coverage | Toggle SHOW DOWNLOADED to confirm the blue box covers your planned area. |

⛔ 500MB download limit per region. If your area exceeds this, download in multiple smaller regions. Always leave extra margin around your planned riding area.

⚠ Tiles survive app updates and reinstalls — stored in external storage. After switching from debug to production build, tiles must be re-downloaded once.

# 9. GPX/KML Track Export

GroupTrack exports your recorded GPS track in GPX or KML format when recording is stopped. The export format is controlled by the toggle in the GEAR panel.

| Format | Description |
| --- | --- |
| GPX | GPS Exchange Format. Compatible with Garmin, Strava, AllTrails, and most navigation apps. |
| KML | Keyhole Markup Language. Compatible with Google Earth, Google Maps, and mapping platforms. |

⚠ V2.4: Export file written to app storage. Share via email/AirDrop targeted for V2.4.2 — direct share sheet integration.

# 10. GEAR Panel — Settings and Build Info

Tap the GEAR icon in the map controls to open the settings panel. The GEAR panel contains map controls, ride settings, and build information.

| Control | Description |
| --- | --- |
| Build stamp | GroupTrack v2.4 — Build YYYYMMDD.HHMM (UTC). Reference this in all issue reports to tester coordinator. |
| Lead lock slider | Adjust lead lock distance threshold 300-1500ft. |
| GPX/KML toggle | Select export format for GPS track recording. |
| FIND AREA | Opens geocoder search for location-based map navigation. |
| DOWNLOAD REGION | Activates rectangle draw tool for offline tile download. |
| SHOW/HIDE DOWNLOADED | Toggle blue coverage overlay showing downloaded tile extent. |
| Offline toggle | Switch between online tile server and local downloaded tiles. |

# 11. GroupTrack Menu — EVENT / RIDE Submenu

Tap the GroupTrack header at the top of the map to open the EVENT / RIDE submenu. This is the navigation hub for ride management and radio configuration.

| Item | Description |
| --- | --- |
| Create a Ride | Create a new ride with name, date, and auto-generated encrypted channel. |
| Transfer Event/Ride Details via Email | Select a ride and send to riders as a .convoy email attachment. |
| Apply Ride Info to Radio | Configure the connected radio with master config or specific ride config. |
| Restore Prior Config Post Ride | Select and restore a saved radio backup from the archive list. |
| Capture Master Config | Read and save the current radio configuration as the new master config. |

# 12. Apply Master Config to Radio

Apply Master Config resets the connected radio to GroupTrack baseline settings. All 47 configuration fields are written in a single operation.

| Field Group | Fields Written |
| --- | --- |
| LoRa | Region (US), Modem Preset (LONG_FAST), Bandwidth, Spread Factor, Coding Rate, Hop Limit (3), TX Power (27dBm), Channel Num (5 = 903MHz) |
| Channel | Channel Name (Convoy), Encryption Key (AES-256 32-byte), Channel ID, Uplink/Downlink enabled |
| Device | Long Name, Node Role, Is Managed, Serial Enabled |
| Position | GPS Enabled, GPS Mode, GPS Update Interval, Attempt Time, Broadcast Interval, Smart Broadcast, Fixed Position, Position Flags |
| Display | Units, Screen Timeout, Auto Brightness, Compass North Top |
| Modules | Telemetry, MQTT, Serial, External Notification, Range Test, Store Forward, Neighbor Info, Detection Sensor, Audio |

⚠ Short Name is NOT written by master config. The radio keeps its existing short name. This prevents accidental node identity changes.

## 12.1 Apply Ride Config

Apply Ride Config writes the master config baseline plus a specific ride's unique channel name and AES-256 encryption key. Used to configure radios for a specific organized ride.

# 13. Capture Master Config from Radio

The Capture screen reads the current radio configuration and saves it as the new master config asset. Used by the GroupTrack developer to update the master config baseline.

| Step | Detail |
| --- | --- |
| 1. Connect radio | Radio must be connected via Bluetooth with correct convoy channel and config. |
| 2. Open Capture | GroupTrack menu → EVENT / RIDE → Capture Master Config. |
| 3. Tap CAPTURE | Reads all config fields from radio. Builds master_config.json and master.cfg binary. |
| 4. Review log | Screen displays all captured values. Verify channel name, PSK, region, LoRa settings. |
| 5. Pull assets | Screen displays exact adb commands using the correct package name for pulling files to repo. |

✅ The capture screen now displays the correct adb pull commands for both debug and production builds automatically.

# 14. Radio Config Archive and Restore

GroupTrack automatically archives the radio configuration before each apply operation. Archived configs can be restored after a ride to return the radio to its pre-ride state.

| Function | Detail |
| --- | --- |
| Auto-archive | Radio config saved automatically before each Apply operation. Timestamped filename. |
| Archive list | GroupTrack menu → Restore Prior Config. Lists all saved configs with date/time. |
| Restore | Select archive entry and tap RESTORE. Config written back to radio. |

# 15. Build Stamp — Version Identification

Every GroupTrack build contains a build stamp visible in the GEAR panel. The build stamp uniquely identifies the exact build for issue reporting.

| Element | Detail |
| --- | --- |
| Display location | GEAR panel — top of settings panel. GroupTrack v2.4 — Build YYYYMMDD.HHMM |
| Format | YYYYMMDD.HHMM in UTC time. Example: 20260405.1246 = April 5 2026 12:46 UTC |
| Issue reporting | Always include build stamp when reporting issues to tester coordinator. Allows developer to identify exact code version. |

# 16. Radio Firmware 2.7 — Known Issue and Workaround

GroupTrack V2.4 Apply Master Config does not write channel and encryption settings to radios running Meshtastic firmware 2.7.x. All other configuration fields (LoRa, GPS, display, modules) apply correctly.

⛔ If your radio runs firmware 2.7.x, channel and PSK will NOT be written by Apply Master Config. You must configure these manually after applying.

## 16.1 Check Your Firmware Version

Connect your radio to the Meshtastic app. Settings → Device → Firmware version. Version 2.6.x is not affected. Version 2.7.x requires the manual workaround below.

## 16.2 Manual Channel Configuration Workaround

| Step | Location | Action |
| --- | --- | --- |
| 1 | Apply Master Config | Apply as normal. All fields except channel/PSK will write correctly. |
| 2 | Meshtastic app | Open Meshtastic app. Settings → Radio Config → Channels. |
| 3 | Primary channel | Set Name to: Convoy |
| 4 | PSK field | Set PSK to: MEJHR0hQQVliRFpVbDAwSW1ocUF5d2Y2S0hobExCaFo= |
| 5 | Save | Tap Save. Radio reboots. Verify channel name shows Convoy in Nodes tab. |

✅ V2.4.1 (Sunday April 12) will fix the 2.7 firmware channel write automatically. After installing V2.4.1 the manual workaround is no longer required.

GroupTrack  |  User Manual V2.4  |  April 5, 2026  |  For Beta Testers