GroupTrack

V2.5 Planning — Updated April 17, 2026

# 1. Package Migration

Current: com.geeksville.mesh.google.debug  |  Target: com.grouptrack.android

- Single renameTo() on tiles/ directory — atomic, instant, no per-file cost

- Target shared external path: /sdcard/GroupTrack/tiles/ — survives package changes

- Migration launcher fires on first run under new package ID

- Do now — before large user tile caches accumulate

# 2. Map Functions — V2.5 Sprint

| ID | Function | File | Notes |
| --- | --- | --- | --- |
| MAP-01 | GPS recording 1s interval | ConvoyGpsService.kt:249 | Change 3000L to 1000L. P1. |
| MAP-02 | Lead cart 5-point broadcast | ConvoyLeadTrackBroadcaster.kt (new) | 86-byte PRIVATE_APP packet. 5 pts x 1s window. No channel utilization change. |
| MAP-03 | KML/GPX route overlay | convoy_map.html + ConvoyScreen.kt | Leaflet omnivore. Android file picker. evaluateJavascript bridge. Toggle show/hide. |
| MAP-04 | Utah UGRC trail overlay | convoy_map.html | Dynamic bbox query on pan. api.arcgis.com. CartoCode=4 filter. |
| MAP-05 | USFS national OHV overlay | convoy_map.html | ALLOWED_TERRA_USE 4/5/6. All 50 states NF land. |
| MAP-07 | Waypoint management | convoy_map.html + ConvoyScreen.kt | Drop pins, name, persist, show on map. |
| MAP-09 | Tile download manager | ConvoyTileDownloader.kt | Select region, download, manage. Integrated with map UI. |
| MAP-10 | Overlay manager | convoy_map.html + ConvoyScreen.kt | Toggle/remove loaded overlays. |

# 3. Open V2.4 Issues

| ID | Issue | Fix Approach |
| --- | --- | --- |
| BLE-01 | BLE scan button missing — ApplicationRegistrationFailed on resume | Reset GATT registration on onResume/onPause lifecycle. Find stale cleanup point. |
| BLE-02 | Radio requires power cycle in some scenarios | Same fix as BLE-01. |
| GPS-01 | Standalone GPS track recording without radio | Use Android GPS when no radio connected. GroupTrack as Gaia/OnX equivalent. |
| GPS-02 | Standalone mode — assign My Cart as lead, track without radio | No radio = My Cart is lead by default. Full track recording and display. |
| DEMO-01 | Remove Live/Demo mode toggle from V3 | V3 backlog — always live mode. |

# 4. Radio — Stay on LONG_FAST

Analysis complete. LONG_FAST with MAP-02 multi-point broadcast = 1-second effective track resolution at full 3-5 mile range, 25% channel utilization. No modem change required.

GroupTrack  |  GroupTrack_V25_Planning_v2  |  April 17, 2026